Qualification Report

OpenCellular - Connect1

Radio Frequency module with Software Defined Radio (RF-SDR)

Revision: 1.0

[16th JAN 2017]

# Table of Contents

Table of Contents	2

1.	Purpose	24

2.	Scope	24

3.	References	24

4.	Device-Under-Test (DUT) Details	24

5.	Qualification Test Condition	25

6.	Qualification Result Summary	26

7.	Tools and Test Equipment	30

8	Power	32

8.1	Voltage accuracy	32

8.1.1	Test ID	32

8.1.2	Purpose	32

8.1.3	Test and Measurement Method	32

8.1.4	Test Condition	32

8.1.5	DUT Sample Information	32

8.1.6	Test Results	33

8.2	Load Regulation	35

8.2.1	Test ID	35

8.2.2	Purpose	35

8.2.3	Test and Measurement Method	35

8.2.4	Test Condition	35

8.2.5	DUT Sample Information	35

8.2.6	Test Results	36

8.3	Line Regulation	38

8.3.1	Test ID	38

8.3.2	Purpose	38

8.3.3	Test and Measurement Method	38

8.3.4	Test Condition	38

8.3.5	DUT Sample Information	38

8.3.6	Test Results	39

8.4	Ripple Noise	43

8.4.1	Test ID	43

8.4.2	Purpose	43

8.4.3	Test and Measurement Method	43

8.4.4	Test Condition	43

8.4.5	DUT Sample Information	43

8.4.6	Test Results	44

8.4.7	Measurement logs	45

8.5	Voltage Control	46

8.5.1	Test ID	46

8.5.2	Purpose	46

8.5.3	Test and Measurement Method	46

8.5.4	Test Condition	46

8.5.5	DUT Sample Information	46

8.5.6	Test Results	46

8.6	Voltage accuracy	47

8.6.1	Test ID	47

8.6.2	Purpose	47

8.6.3	Test and Measurement Method	47

8.6.4	Test Condition	47

8.6.5	DUT Sample Information	47

8.6.6	Test Results	48

8.7	Voltage Control	50

8.7.1	Test ID	50

8.7.2	Purpose	50

8.7.3	Test and Measurement Method	50

8.7.4	Test Condition	50

8.7.5	DUT Sample Information	50

8.7.6	Test Results	50

8.8	Current consumption	51

8.8.1	Test ID	51

8.8.2	Purpose	51

8.8.3	Test and Measurement Method	51

8.8.4	Test Condition	51

8.8.5	DUT Sample Information	51

8.8.6	Test Results	52

9	Clock	53

9.1	Clock output level	53

9.1.1	Test ID	53

9.1.2	Purpose	53

9.1.3	Test and Measurement Method	53

9.1.4	Test Condition	53

9.1.5	DUT Sample Information	53

9.1.6	Test Results	53

9.2	Frequency and Frequency accuracy	54

9.2.1	Test ID	54

9.2.2	Purpose	54

9.2.3	Test and Measurement Method	54

9.2.4	Test Condition	54

9.2.5	DUT Sample Information	54

9.2.6	Test Results	54

9.3	Phase noise	55

9.3.1	Test ID	55

9.3.2	Purpose	55

9.3.3	Test and Measurement Method	55

9.3.4	Test Condition	55

9.3.5	DUT Sample Information	55

9.3.6	Test Results	56

9.3.7	Test and Measurement Logs	57

9.4	Lock time	58

9.4.1	Test ID	58

9.4.2	Purpose	58

9.4.3	Test and Measurement Method	58

9.4.4	Test Condition	58

9.4.5	DUT Sample Information	58

9.4.6	Test Results	58

9.5	Clock Duty Cycle	59

9.5.1	Test ID	59

9.5.2	Purpose	59

9.5.3	Test and Measurement Method	59

9.5.4	Test Condition	59

9.5.5	DUT Sample Information	59

9.5.6	Test Results	59

9.5.7	Test and Measurement Logs	60

9.6	Jitter	61

9.6.1	Test ID	61

9.6.2	Purpose	61

9.6.3	Test and Measurement Method	61

9.6.4	Test Condition	61

9.6.5	DUT Sample Information	61

9.6.6	Test Results	61

9.6.7	Test and Measurement Logs	62

9.7	Clock Duty Cycle	63

9.7.1	Test ID	63

9.7.2	Purpose	63

9.7.3	Test and Measurement Method	63

9.7.4	Test Condition	63

9.7.5	DUT Sample Information	63

9.7.6	Test Results	63

9.7.7	Test and Measurement Logs	64

9.8	Jitter	65

9.8.1	Test ID	65

9.8.2	Purpose	65

9.8.3	Test and Measurement Method	65

9.8.4	Test Condition	65

9.8.5	DUT Sample Information	65

9.8.6	Test Results	65

9.8.7	Test and Measurement Logs	66

10	FPGA	67

10.1	Artix – 7 –Boot configuration	67

10.1.1	Test ID	67

10.1.2	Purpose	67

10.1.3	Test and Measurement Method	67

10.1.4	Test Condition	67

10.1.5	DUT Sample Information	67

10.1.6	Test Results	67

10.1.7	Test and Measurement Logs	68

10.2	Artix – 7 –Power sequence	69

10.2.1	Test ID	69

10.2.2	Purpose	69

10.2.3	Test and Measurement Method	69

10.2.4	Test Condition	69

10.2.5	DUT Sample Information	69

10.2.6	Test Results	69

10.2.7	Test and Measurement Logs	70

10.3	FX3 – SPI –Electrical validation	71

10.3.1	Test ID	71

10.3.2	Purpose	71

10.3.3	Test and Measurement Method	71

10.3.4	Test Condition	71

10.3.5	DUT Sample Information	71

10.3.6	Test Results	72

10.3.7	Test and Measurement Logs	72

10.4	FX3 – SPI –Functional validation	73

10.4.1	Test ID	73

10.4.2	Purpose	73

10.4.3	Test and Measurement Method	73

10.4.4	Test Condition	73

10.4.5	DUT Sample Information	73

10.4.6	Test Results	73

10.4.7	Test and Measurement Logs	73

10.5	AD9361 – SPI-Electrical validation/Signal integrity	74

10.5.1	Test ID	74

10.5.2	Purpose	74

10.5.3	Test and Measurement Method	74

10.5.4	Test Condition	74

10.5.5	DUT Sample Information	74

10.5.6	Test Results	75

10.5.7	Test and Measurement Logs	75

10.5.8	Purpose	76

10.5.9	Test and Measurement Method	76

10.5.10	Test Condition	76

10.5.11	DUT Sample Information	76

10.5.12	Test Results	77

10.5.13	Test and Measurement Logs	78

10.6	AD9361 – SPI-Functional validation	79

10.6.1	Test ID	79

10.6.2	Purpose	79

10.6.3	Test and Measurement Method	79

10.6.4	Test Condition	79

10.6.5	DUT Sample Information	79

10.6.6	Test Results	79

10.6.7	Test and Measurement Logs	80

10.7	FX3 – GPIF Control – Electrical validation	81

10.7.1	Test ID	81

10.7.2	Purpose	81

10.7.3	Test and Measurement Method	81

10.7.4	Test Condition	81

10.7.5	DUT Sample Information	81

10.7.6	Test Results	82

10.7.7	Test and Measurement Logs	82

10.8	FX3 – GPIF Control – Functional validation	83

10.8.1	Test ID	83

10.8.2	Purpose	83

10.8.3	Test and Measurement Method	83

10.8.4	Test Condition	83

10.8.5	DUT Sample Information	83

10.8.6	Test Results	83

10.8.7	Test and Measurement Logs	84

10.9	FX3 – GPIF Data – Electrical validation	85

10.9.1	Test ID	85

10.9.2	Purpose	85

10.9.3	Test and Measurement Method	85

10.9.4	Test Condition	85

10.9.5	DUT Sample Information	85

10.9.6	Test Results	86

10.9.7	Test and Measurement Logs	87

10.10	FX3 – GPIF Data – Functional validation	88

10.10.1	Test ID	88

10.10.2	Purpose	88

10.10.3	Test and Measurement Method	88

10.10.4	Test Condition	88

10.10.5	DUT Sample Information	88

10.10.6	Test Results	88

10.10.7	Test and Measurement Logs	89

10.11	AD9361 – Control- Electrical validation	90

10.11.1	Test ID	90

10.11.2	Purpose	90

10.11.3	Test and Measurement Method	90

10.11.4	Test Condition	90

10.11.5	DUT Sample Information	90

10.11.6	Test Results	90

10.11.7	Test and Measurement Logs	91

10.12	AD9361 – Control- Functional validation	92

10.12.1	Test ID	92

10.12.2	Purpose	92

10.12.3	Test and Measurement Method	92

10.12.4	Test Condition	92

10.12.5	DUT Sample Information	92

10.12.6	Test Results	92

10.12.7	Test and Measurement Logs	93

10.13	AD9361 – Data- Electrical validation	94

10.13.1	Test ID	94

10.13.2	Purpose	94

10.13.3	Test and Measurement Method	94

10.13.4	Test Condition	94

10.13.5	DUT Sample Information	94

10.13.6	Test Results	95

10.13.7	Test and Measurement Logs	96

10.14	AD9361 – Data- Functional validation	97

10.14.1	Test ID	97

10.14.2	Purpose	97

10.14.3	Test and Measurement Method	97

10.14.4	Test Condition	97

10.14.5	DUT Sample Information	97

10.14.6	Test Results	97

10.14.7	Test and Measurement Logs	98

11	FX3	99

11.1	FX3 (CYUSB3014)-Configuration	99

11.1.1	Test ID	99

11.1.2	Purpose	99

11.1.3	Test and Measurement Method	99

11.1.4	Test Condition	99

11.1.5	DUT Sample Information	99

11.1.6	Test Results	99

11.2	EEPROM (24LC256) – I2C –Electrical validation	100

11.2.1	Test ID	100

11.2.2	Purpose	100

11.2.3	Test and Measurement Method	100

11.2.4	Test Condition	100

11.2.5	DUT Sample Information	100

11.2.6	Test Results	101

11.2.7	Test and Measurement Logs	101

11.3	EEPROM (24LC256) – I2C –Functional validation	102

11.3.1	Test ID	102

11.3.2	Purpose	102

11.3.3	Test and Measurement Method	102

11.3.4	Test Condition	102

11.3.5	DUT Sample Information	102

11.3.6	Test Results	102

11.3.7	Test and Measurement Logs	102

11.4	Functional validation of Debug USB Switch – USB2.0 from FX3	103

11.4.1	Test ID	103

11.4.2	Purpose	103

11.4.3	Test and Measurement Method	103

11.4.4	Test Condition	103

11.4.5	DUT Sample Information	103

11.4.6	Test Results	103

11.4.7	Test and Measurement Logs	103

11.5	Functional validation of Debug USB Switch – USB3.0 from FX3	104

11.5.1	Test ID	104

11.5.2	Purpose	104

11.5.3	Test and Measurement Method	104

11.5.4	Test Condition	104

11.5.5	DUT Sample Information	104

11.5.6	Test Results	104

11.5.7	Test and Measurement Logs	104

12	RF/Transceiver      (AD9361) – Pipe1	105

12.1	Maximum Output Power from AD9361- Pipe 1	105

12.1.1	Test ID	105

12.1.2	Purpose	105

12.1.3	Test and Measurement Method	105

12.1.4	Test Condition	105

12.1.5	DUT Sample Information	105

12.1.6	Test Results	106

12.2	Transmit Power Control from AD9361-Pipe1	107

12.2.1	Test ID	107

12.2.2	Purpose	107

12.2.3	Test and Measurement Method	107

12.2.4	Test Condition	107

12.2.5	DUT Sample Information	107

12.2.6	Test Results	108

12.3	Modulation Accuracy –TRx – Pipe 1	110

12.3.1	Test ID	110

12.3.2	Purpose	110

12.3.3	Test and Measurement Method	110

12.3.4	Test Condition	110

12.3.5	DUT Sample Information	110

12.3.6	Test Results	111

12.3.7	Test and Measurement Logs	112

12.4	Output RF Spectrum- i) Adjacent channel power-TRx Pipe 1	113

12.4.1	Test ID	113

12.4.2	Purpose	113

12.4.3	Test and Measurement Method	113

12.4.4	Test Condition	113

12.4.5	DUT Sample Information	113

12.4.6	Test Results	114

12.4.7	Test and Measurement Logs	115

ii) Spectrum due to switching- TRx Pipe 1	116

12.4.8	Test ID	116

12.4.9	Purpose	116

12.4.10	Test and Measurement Method	116

12.4.11	Test Condition	116

12.4.12	DUT Sample Information	116

12.4.13	Test Results	117

12.4.14	Test and Measurement Logs	118

12.5	Carrier leakage - Pipe1	119

12.5.1	Test ID	119

12.5.2	Purpose	119

12.5.3	Test and Measurement Method	119

12.5.4	Test Condition	119

12.5.5	DUT Sample Information	119

12.5.6	Test Results	120

12.5.7	Test and Measurement Logs	120

13	RF/Transceiver      (AD9361) – Pipe2	121

13.1	Maximum Output Power from AD9361-Pipe2	121

13.1.1	Test ID	121

13.1.2	Purpose	121

13.1.3	Test and Measurement Method	121

13.1.4	Test Condition	121

13.1.5	DUT Sample Information	121

13.1.6	Test Results	122

13.2	Transmit Power Control from AD9361-Pipe2	123

13.2.1	Test ID	123

13.2.2	Purpose	123

13.2.3	Test and Measurement Method	123

13.2.4	Test Condition	123

13.2.5	DUT Sample Information	123

13.2.6	Test Results	124

13.3	Modulation Accuracy –TRx – Pipe 2	126

13.3.1	Test ID	126

13.3.2	Purpose	126

13.3.3	Test and Measurement Method	126

13.3.4	Test Condition	126

13.3.5	DUT Sample Information	126

13.3.6	Test Results	126

13.3.7	Test and Measurement Logs	127

13.4	AD9361 Local Oscillator lock detect- Pipe 1&2	128

13.4.1	Test ID	128

13.4.2	Purpose	128

13.4.3	Test and Measurement Method	128

13.4.4	Test Condition	128

13.4.5	DUT Sample Information	128

13.4.6	Test Results	128

13.5	Output RF Spectrum- i) Adjacent channel power-TRx Pipe 2	129

13.5.1	Test ID	129

13.5.2	Purpose	129

13.5.3	Test and Measurement Method	129

13.5.4	Test Condition	129

13.5.5	DUT Sample Information	129

13.5.6	Test Results	130

13.5.7	Test and Measurement Logs	131

ii) Spectrum due to switching- TRx Pipe 2	132

13.5.8	Test ID	132

13.5.9	Purpose	132

13.5.10	Test and Measurement Method	132

13.5.11	Test Condition	132

13.5.12	DUT Sample Information	132

13.5.13	Test Results	133

13.5.14	Test and Measurement Logs	134

13.6	Carrier leakage – Pipe2	135

13.6.1	Test ID	135

13.6.2	Purpose	135

13.6.3	Test and Measurement Method	135

13.6.4	Test Condition	135

13.6.5	DUT Sample Information	135

13.6.6	Test Results	136

13.6.7	Test and Measurement Logs	136

14	TX pipe – 1	137

14.1	Gain-Pipe1	137

14.1.1	Test ID	137

14.1.2	Purpose	137

14.1.3	Test and Measurement Method	137

14.1.4	Test Condition	137

14.1.5	DUT Sample Information	137

14.1.6	Test Results (Rev-A)	138

14.2	Attenuation and Attenuation step- TX Pipe1	139

14.2.1	Test ID	139

14.2.2	Purpose	139

14.2.3	Test and Measurement Method	139

14.2.4	Test Condition	139

14.2.5	DUT Sample Information	139

14.2.6	Test Results	140

14.3	Output Power- TX Pipe 1	142

14.3.1	Test ID	142

14.3.2	Purpose	142

14.3.3	Test and Measurement Method	142

14.3.4	Test Condition	142

14.3.5	DUT Sample Information	142

14.3.6	Test Results	142

14.4	RF power detection – TX Pipe1	143

14.4.1	Test ID	143

14.4.2	Purpose	143

14.4.3	Test and Measurement Method	143

14.4.4	Test Condition	143

14.4.5	DUT Sample Information	143

14.4.6	Test Results	144

15	TX pipe – 2	145

15.1	Gain-Pipe2	145

15.1.1	Test ID	145

15.1.2	Purpose	145

15.1.3	Test and Measurement Method	145

15.1.4	Test Condition	145

15.1.5	DUT Sample Information	145

15.1.6	Test Results (Rev-A)	146

15.2	Attenuation and Attenuation step- TX Pipe2	147

15.2.1	Test ID	147

15.2.2	Purpose	147

15.2.3	Test and Measurement Method	147

15.2.4	Test Condition	147

15.2.5	DUT Sample Information	147

15.2.6	Test Results	148

15.3	Output Power- TX Pipe 2	150

15.3.1	Test ID	150

15.3.2	Purpose	150

15.3.3	Test and Measurement Method	150

15.3.4	Test Condition	150

15.3.5	DUT Sample Information	150

15.3.6	Test Results	150

15.4	RF power detection – Tx Pipe 2	151

15.4.1	Test ID	151

15.4.2	Purpose	151

15.4.3	Test and Measurement Method	151

15.4.4	Test Condition	151

15.4.5	DUT Sample Information	151

15.4.6	Test Results	152

16	RX pipe – 1	153

16.1	Noise Figure and Gain – Rx Pipe-1	153

16.1.1	Test ID	153

16.1.2	Purpose	153

16.1.3	Test and Measurement Method	153

16.1.4	Test Condition	153

16.1.5	DUT Sample Information	153

16.1.6	Test Results for Noise Figure	154

16.1.7	Test Results for Gain	154

16.1.8	Test and Measurement Logs	155

16.2	Attenuation and Attenuation step- Rx Pipe1	156

16.2.1	Test ID	156

16.2.2	Purpose	156

16.2.3	Test and Measurement Method	156

16.2.4	Test Condition	156

16.2.5	DUT Sample Information	156

16.2.6	Test Results	157

17	RX pipe -2	159

17.1	Noise Figure and Gain – Rx Pipe-2	159

17.1.1	Test ID	159

17.1.2	Purpose	159

17.1.3	Test and Measurement Method	159

17.1.4	Test Condition	159

17.1.5	DUT Sample Information	159

17.1.6	Test Results for Noise Figure	160

17.1.7	Test Results for Gain	160

17.1.8	Test and Measurement Logs	161

17.2	Attenuation and Attenuation step- Rx Pipe2	162

17.2.1	Test ID	162

17.2.2	Purpose	162

17.2.3	Test and Measurement Method	162

17.2.4	Test Condition	162

17.2.5	DUT Sample Information	162

17.2.6	Test Results	163

18	Transmitter _Chain 1	165

18.1	i) Output Power- TX Chain 1	165

18.1.1	Test ID	165

18.1.2	Purpose	165

18.1.3	Test and Measurement Method	165

18.1.4	Test Condition	165

18.1.5	DUT Sample Information	165

18.1.6	Test Results	165

ii)	Power Vs Time – TX Chain 1	166

18.1.7	Test ID	166

18.1.8	Purpose	166

18.1.9	Test and Measurement Method	166

18.1.10	Test Condition	166

18.1.11	DUT Sample Information	166

18.1.12	Test Results	166

18.1.13	Test and Measurement Logs	166

18.2	Static power control – TX chain1	167

18.2.1	Test ID	167

18.2.2	Purpose	167

18.2.3	Test and Measurement Method	167

18.2.4	Test Condition	167

18.2.5	DUT Sample Information	167

18.2.6	Test Results	168

18.3	Modulation Accuracy for TX – Chain 1	170

18.3.1	Test ID	170

18.3.2	Purpose	170

18.3.3	Test and Measurement Method	170

18.3.4	Test Condition	170

18.3.5	DUT Sample Information	170

18.3.6	Test Results	171

18.3.7	Test and Measurement Logs	172

18.4	Output RF Spectrum- i) Adjacent channel power-TX Chain 1	173

18.4.1	Test ID	173

18.4.2	Purpose	173

18.4.3	Test and Measurement Method	173

18.4.4	Test Condition	173

18.4.5	DUT Sample Information	173

18.4.6	Test Results	174

18.4.7	Test and Measurement Logs	175

ii) Spectrum due to switching- TX Chain 1	176

18.4.8	Test ID	176

18.4.9	Purpose	176

18.4.10	Test and Measurement Method	176

18.4.11	Test Condition	176

18.4.12	DUT Sample Information	176

18.4.13	Test Results	177

18.4.14	Test and Measurement Logs	178

18.5	Spurious Emissions – TX chain1	179

18.5.1	Test ID	179

18.5.2	Purpose	179

18.5.3	Test and Measurement Method	179

18.5.4	Test Condition	179

18.5.5	DUT Sample Information	179

18.5.6	Test Results	180

19	Transmitter _Chain 2	182

19.1	i) Output Power- TX Chain 2	182

19.1.1	Test ID	182

19.1.2	Purpose	182

19.1.3	Test and Measurement Method	182

19.1.4	Test Condition	182

19.1.5	DUT Sample Information	182

19.1.6	Test Results	182

ii)	Power Vs Time – TX Chain 2	183

19.1.7	Test ID	183

19.1.8	Purpose	183

19.1.9	Test and Measurement Method	183

19.1.10	Test Condition	183

19.1.11	DUT Sample Information	183

19.1.12	Test Results	183

19.1.13	Test and Measurement Logs	183

19.2	Static power control – TX chain2	184

19.2.1	Test ID	184

19.2.2	Purpose	184

19.2.3	Test and Measurement Method	184

19.2.4	Test Condition	184

19.2.5	DUT Sample Information	184

19.2.6	Test Results	185

19.3	Modulation Accuracy for TX – Chain 2	187

19.3.1	Test ID	187

19.3.2	Purpose	187

19.3.3	Test and Measurement Method	187

19.3.4	Test Condition	187

19.3.5	DUT Sample Information	187

19.3.6	Test Results	188

19.3.7	Test and Measurement Logs	189

19.4	Output RF Spectrum- i) Adjacent channel power-Tx Chain 2	190

19.4.1	Test ID	190

19.4.2	Purpose	190

19.4.3	Test and Measurement Method	190

19.4.4	Test Condition	190

19.4.5	DUT Sample Information	190

19.4.6	Test Results	191

19.4.7	Test and Measurement Logs	192

ii) Spectrum due to switching- TX Chain 2	193

19.4.8	Test ID	193

19.4.9	Purpose	193

19.4.10	Test and Measurement Method	193

19.4.11	Test Condition	193

19.4.12	DUT Sample Information	193

19.4.13	Test Results	194

19.4.14	Test and Measurement Logs	195

19.5	Spurious Emissions – TX chain2	196

19.5.1	Test ID	196

19.5.2	Purpose	196

19.5.3	Test and Measurement Method	196

19.5.4	Test Condition	196

19.5.5	DUT Sample Information	196

19.5.6	Test Results	197

20	Revision History	199

# Purpose

The purpose of this document is to capture test data for Radio-Frequency module with Software-Defined-Radio (RF-SDR) as part of Open Cellular Base Transceiver Station (BTS). The document is intended to provide a formal report of measured and validated parameters to qualify RF-SDR module as part of design validation testing to ensure consistent and reliable operation across all supported operating and environmental conditions.

# Scope

Scope of this document is to qualify different sections as mentioned below:

- Power Source section which includes Volatge regualators, FPGA PMIC

- Clock section which includes VCTCXO, PLL

- FPGA and FX3 section

- Transceiver Section (AD9361)

- Transmitter and Receiver Sections

# References

- Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification https://github.com/markhor/OpenCellular

- Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Plan https://github.com/markhor/OpenCellular

- RF_SDR – datasheets

https://github.com/markhor/OpenCellular

# Device-Under-Test (DUT) Details

- System	: Open cellular Connect -1

- Sub-system	: RF-SDR

- Hardware version	: Life – 1 & Life -2

- Software version	: The git versions as follows

  - Openbsc            :   5085e0b

  - Osmo-trx           :   2e5e2c5

  - Uhd                     :    f70dd85

- Sample Count	:  01

# Qualification Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

# Qualification Result Summary

|  | Sub-system | Function | Test cases/specification | Priority | Status |
| --- | --- | --- | --- | --- | --- |
| Test ID |  |  |  |  |  |
| Pwr1.1 | Power | 1. Conversion from 12V input from GBC board to 5.7, 3.7, 1.8, 1.2, 3.3,1 and 5.1V output using switching regulators. | Voltage Accuracy | P0 | Pass |
| Pwr1.2 |  |  | Load regulation | P0 | Pass |
| Pwr1.3 |  |  | Line regulation | P0 | Pass |
| Pwr1.4 |  |  | Ripple Noise | P1 | Pass |
| Pwr1.5 |  |  | Voltage Control | P0 | Pass |
| Pwr1.6 |  | 2. Converting and regulating from switching regulators out to voltages required by various devices on RF_SDR section using LDO’s. | Voltage Accuracy | P0 | Pass |
| Pwr1.7 |  |  | Voltage Control | P0 | Pass |
| Pwr1.8 |  | 3. Read current flow via current sensor circuit. | Current consumption | P0 | Pass |
| Clk 1.1 | Clock | To synchronize with the GBC system clock and generate clock frequency required for transceiver IC | Clock output level |  | Pass |
| Clk 1.2 |  |  | Frequency |  | Pass |
| Clk 1.3 |  |  | Frequency accuracy |  | Pass |
| Clk 1.4 |  |  | Phase noise |  | Fail |
| Clk 1.5 |  |  | Lock time |  | Pass |
| Clk 1.6 |  | To synchronize with the GBC system clock and generate clock frequency required for codec FPGA | Clock Duty Cycle | P1 | Pass |
| Clk 1.7 |  |  | Jitter | P1 | Pass |
| Clk 1.8 |  | Clock from AD9361 out to FPGA and other digital clocks | Clock Duty Cycle | P4 | Pass |
| Clk 1.9 |  |  | Jitter | P4 | Pass |
| FPGA 1.1.1 | FPGA | Artix-7 | Boot configuration | P0 | Pass |
| FPGA 1.1.2 |  |  | Power sequence | P0 | Pass |
| FPGA 1.2.1 |  | FX3 - SPI | Electrical validation | P0 | Pass |
| FPGA 1.2.2 |  |  | Functional validation | P1 | Pass |
| FPGA 1.3.1 |  | AD9361 - SPI | Electrical validation | P0 | Fail |
| FPGA 1.3.2 |  |  | Functional validation | P1 | Pass |
| FPGA 1.4.1 |  | FX3 - GPIF:Control | Electrical validation | P0 | Pass |
| FPGA 1.4.2 |  |  | Functional validation | P1 | Pass |
| FPGA 1.5.1 |  | FX3 - GPIF:Data | Electrical validation | P0 | Pass |
| FPGA 1.5.2 |  |  | Functional validation | P1 | Pass |
| FPGA 1.6.1 |  | AD9361 - Control | Electrical validation | P0 | Pass |
| FPGA 1.6.2 |  |  | Functional validation | P1 | Pass |
| FPGA 1.7.1 |  | AD9361 - Data | Electrical validation | P0 | Fail |
| FPGA 1.7.2 |  |  | Functional validation | P1 | Pass |
| FX3 1.1 | FX3 | FX3 | Configuration | P0 | Pass |
| FX3 1.2.1 |  | EEPROM-I2C | Electrical validation | P0 | Pass |
| FX3 1.2.2 |  |  | Functional validation | P0 | Pass |
| FX3 1.3.1 |  | Debug USB Switch - USB2.0 | Functional validation | P0 | Pass |
| FX3 1.4.1 |  | Debug USB Switch - USB 3.0 | Functional validation | P0 | Pass |
| TRX 1.1 | RF/Transceiver (AD9361) - Pipe1 | AD9361(Transceiver IC)is used to convert base band data to RF on transmit side and RF to base band on receive side. | Maximum Output Power |  | Pass |
| TRX 1.2 |  |  | Transmit power control-ATTENUATION RANGE |  | Pass |
| TRX 1.3 |  |  | Modulation Accuracy | P2 | Pass |
| TRX 1.4 |  |  | Carrier Leakage | P2 | Pass |
| TRX 1.5 |  |  | LO Lock Detect |  | Pass |
| TRX 1.6 |  |  | Output RF spectrum i). Adjacent channel power ii). Spectrum due to switching | P2 | Fail |
| TRX 1.7 |  |  | Receiver sensitivity | P1 | Open |
| TRX 1.8 |  |  | Maximum Input signal | P1 | Open |
| TRX 2.1 | RF/Transceiver (AD9361) - Pipe2 |  | Maximum Output Power |  | Pass |
| TRX 2.2 |  |  | Transmit power control-ATTENUATION RANGE |  | Pass |
| TRX 2.3 |  |  | Modulation Accuracy | P2 | Pass |
| TRX 2.4 |  |  | Carrier Leakage | P2 | Pass |
| TRX 2.5 |  |  | LO Lock Detect |  | Pass |
| TRX 2.6 |  |  | Output RF spectrum i). Adjacent channel power ii). Spectrum due to switching | P2 | Fail |
| TRX 2.7 |  |  | Receiver sensitivity | P1 | Open |
| TRX 2.8 |  |  | Maximum Input signal | P1 | Open |
| TX_P 1.1 | TX pipe - 1 | TX pipe is used to amplify and control TX signal | Gain |  | Pass |
| TX_P 1.2 |  |  | Attenuation (part of TIVA I2C testing) |  | Pass |
| TX_P 1.3 |  |  | Attenuation step (part of TIVA I2C testing) |  | Pass |
| TX_P 1.4 |  |  | Output Power |  | Pass |
| TX_P 1.5 |  |  | RF power detection | P0 | Pass |
| TX_P 2.1 | TX pipe - 2 |  | Gain |  | Pass |
| TX_P 2.2 |  |  | Attenuation (part of TIVA I2C testing) |  | Pass |
| TX_P 2.3 |  |  | Attenuation step (part of TIVA I2C testing) |  | Pass |
| TX_P 2.4 |  |  | Output Power |  | Pass |
| TX_P 2.5 |  |  | RF power detection | P0 | Pass |
| RX_P 1.1 | RX pipe - 1 | RX pipe is used to amplify and control RX signal | Noise Figure | P0 | Fail |
| RX_P 1.2 |  |  | Gain |  | Fail |
| RX_P 1.3 |  |  | Attenuation (part of TIVA I2C testing) |  | Pass |
| RX_P 1.4 |  |  | Attenuation step (part of TIVA I2C testing) |  | Pass |
| RX_P 2.1 | RX pipe -2 |  | Noise Figure | P0 | Fail |
| RX_P 2.2 |  |  | Gain |  | Fail |
| RX_P 2.3 |  |  | Attenuation (part of TIVA I2C testing) |  | Pass |
| RX_P 2.4 |  |  | Attenuation step (part of TIVA I2C testing) |  | Pass |
|  | Transmitter _Chain 1 | To transmit and receive GSM signals with baseband data from Linux PC. | TX Subsystem along with AD9361 with GBC |  |  |
| TX_C 1.2 |  |  | i) Output Power and Tolerance ii) RF carrier power versus time | P0 | Pass |
| TX_C 1.3 |  |  | Static Power Control | P0 | Pass |
| TX_C 1.4 |  |  | Modulation accuracy | P2 | Pass |
| TX_C 1.5 |  |  | Output RF spectrum i). Adjacent channel power ii). Spectrum due to switching | P2 | Fail |
| TX_C 1.6 |  |  | Spurious Emissions i). Tx and Rx band spurious ii). Cross-band spurious iii). Out-of-band spurious | P0 | Fail |
|  | Transmitter _Chain 2 |  | TX Subsystem along with AD9361 with GBC |  |  |
| TX_C 2.2 |  |  | i) Output Power and Tolerance ii) RF carrier power versus time | P0 | Pass |
| TX_C 2.3 |  |  | Static Power Control | P0 | Pass |
| TX_C 2.4 |  |  | Modulation accuracy | P2 | Pass |
| TX_C 2.5 |  |  | Output RF spectrum i). Adjacent channel power ii). Spectrum due to switching | P2 | Fail |
| TX_C 2.6 |  |  | Spurious Emissions i). Tx and Rx band spurious ii). Cross-band spurious iii). Out-of-band spurious | P0 | Fail |
| RX_C 1.1 | Receiver Chain 1 |  | Reference Sensitivity Level | P1 | Open |
| RX_C 2.1 | Receiver Chain 2 |  | Reference Sensitivity Level | P1 | Open |

# Tools and Test Equipment

| Test (Sub-System) | Tools and Test Equipment | Model and Version Information |
| --- | --- | --- |
| Power | DC Power Supply | RIGOL DP832 |
|  | Electronic Load | KMO64 |
|  | Oscilloscope | MSO9404A |
| Clock | DC Power Supply | RIGOL DP832 |
|  | PXIe chassis | M9381A |
|  | RF Cables | SMA Male to SMA Male |
|  | Pig tail SMA Cables | One end SMA Female connector and another end open cable |
|  | Linux PC | TBD |
|  | Oscilloscope | MSO9404A |
| FPGA | DC Power Supply | RIGOL DP832 |
|  | Oscilloscope | MSO9404A |
|  | Linux PC | TBD |
| FX3 | DC Power Supply | RIGOL DP832 |
|  | Oscilloscope | MSO9404A |
|  | Linux PC | TBD |
| RF/Transceiver (AD9361) – Pipe1 | DC Power Supply | RIGOL DP832 |
|  | PXIe chassis | M9381A |
|  | RF Cables | SMA Male to SMA Male |
|  | Pig tail SMA Cables | One end SMA Female connector and another end open cable |
|  | Linux PC | TBD |
|  | Oscilloscope | MSO9404A |
| RF/Transceiver (AD9361) – Pipe2 | DC Power Supply | RIGOL DP832 |
|  | PXIe chassis | M9381A |
|  | RF Cables | SMA Male to SMA Male |
|  | Pig tail SMA Cables | One end SMA Female connector and another end open cable |
|  | Oscilloscope | MSO9404A |
|  | Linux PC | TBD |
| TX pipe – 1 | DC Power Supply | RIGOL DP832 |
|  | PXIe chassis | M9381A |
|  | RF Cables | SMA Male to SMA Male |
|  | Pig tail SMA Cables | One end SMA Female connector and another end open cable |
| TX pipe – 2 | DC Power Supply | RIGOL DP832 |
|  | PXIe chassis | M9381A |
|  | RF Cables | SMA Male to SMA Male |
|  | Pig tail SMA Cables | One end SMA Female connector and another end open cable |
| RX pipe – 1 | Noise Source | HP – 346B |
|  | Signal Analyzer | Keysight N9020A |
|  | RF Cables | SMA(F) to Switch type cable |
|  |  | BNC to BNC cable |
| RX pipe -2 | Noise Source | HP – 346B |
|  | Signal Analyzer | Keysight N9020A |
|  | RF Cables | SMA(F) to Switch type cable |
|  |  | BNC to BNC cable |
| Transmitter _Chain 1 | DC Power Supply | RIGOL DP832 |
|  | PXIe chassis | M9381A |
|  | RF Cables | SMA Male to SMA Male |
|  | Attenuator | 30dB |
| Transmitter _Chain 2 | DC Power Supply | RIGOL DP832 |
|  | PXIe chassis | M9381A |
|  | RF Cables | SMA Male to SMA Male |
|  | Attenuator | 30dB |
| Receiver Chain 1 |  |  |
| Receiver Chain 2 |  |  |

# Power

# Voltage accuracy

# Test ID

Pwr1.1

# Purpose

The purpose of the test case is to measure the output voltage of switching regulators and to ensure that these voltages are in specified limits.

# Test and Measurement Method

Refer to section 3.1.1 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Full

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0004

# Test Results

The measured output voltage accuracy of switching regulators are within 2% of expected voltage

U3500 (12V to 5.7V):

| Voltage Accuracy for U3500 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply Voltage (V) | Load Current (A) | Output Current (A) | Output Voltage (V) | Specification(V) |  | Design margin (%) | Result |
|  |  |  |  |  |  | Min | Max |  |  |
| 1 | Pwr1.1 | 11.4 | 4 | 2.184 | 5.795 | 5.586 | 5.814 | -0.326797386 | PASS |
| 2 | Pwr1.1 | 12 | 4 | 2.072 | 5.796 | 5.586 | 5.814 | -0.309597523 | PASS |
| 3 | Pwr1.1 | 12.6 | 4 | 1.969 | 5.792 | 5.586 | 5.814 | -0.378396973 | PASS |

| Voltage Accuracy for U3501 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply Voltage (V) | Load Current (A) | Output Current (A) | Output Voltage (V) | Specification(V) |  | Design margin (%) | Result |
|  |  |  |  |  |  | Min | Max |  |  |
| 1 | Pwr1.1 | 11.4 | 4 | 2.153 | 5.713 | 5.586 | 5.814 | -1.737186103 | PASS |
| 2 | Pwr1.1 | 12 | 4 | 2.043 | 5.717 | 5.586 | 5.814 | -1.668386653 | PASS |
| 3 | Pwr1.1 | 12.6 | 4 | 1.942 | 5.714 | 5.586 | 5.814 | -1.71998624 | PASS |

U3501 (12V to 5.7V):

U4000 (12V to 3.7V):

| Voltage Accuracy for U4000 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply Voltage (V) | Load Current (A) | Output Current (A) | Output Voltage (V) | Specification(V) |  | Design margin (%) | Result |
|  |  |  |  |  |  | Min | Max |  |  |
| 1 | Pwr1.1 | 11.4 | 4 | 1.402 | 3.741 | 3.626 | 3.774 | -0.874403816 | PASS |
| 2 | Pwr1.1 | 12 | 4 | 1.338 | 3.741 | 3.626 | 3.774 | -0.874403816 | PASS |
| 3 | Pwr1.1 | 12.6 | 4 | 1.274 | 3.739 | 3.626 | 3.774 | -0.927397986 | PASS |

U3400 (3.7V to 1.8V):

| Voltage Accuracy for U3400 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply Voltage (V) | Load Current (A) | Output Current (A) | Output Voltage (V) | Specification(V) |  | Design margin (%) | Result |
|  |  |  |  |  |  | Min | Max |  |  |
| 1 | Pwr1.1 | 3.6 | 500 | 0.261 | 1.81 | 1.773 | 1.827 | -0.930487137 | PASS |
| 2 | Pwr1.1 | 3.7 | 500 | 0.25 | 1.815 | 1.773 | 1.827 | -0.65681445 | PASS |
| 3 | Pwr1.1 | 3.8 | 500 | 0.248 | 1.817 | 1.773 | 1.827 | -0.547345375 | PASS |

NOTE:  Refer to section 8.4.7 for measurement logs of switching regulators for voltage accuracy.

# Load Regulation

# Test ID

Pwr1.2

# Purpose

The purpose of this test case is to check the capability of switching regulators to maintain a constant output voltage over changes in the load.

# Test and Measurement Method

Refer to section 3.1.2 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage -12V DC

System/Test Load: Min-Typical-Max

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0004

# Test Results

The output voltage accuracy of regulators are within 2% of expected voltage under various load conditions.

U3500 (12V to 5.7V):

| Load Regulation ( Input Voltage – 12V) for U3500 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Load Current (A) | Output Current (A) | Output Voltage (V) (avg) | Specification(V) |  | Design margin (%) | Result |
|  |  |  |  |  | Min | Max |  |  |
| 1 | Pwr1.2 | 1 | 0.494 | 5.788 | 5.586 | 5.814 | -0.447196422 | PASS |
| 2 | Pwr1.2 | 2 | 0.998 | 5.791 | 5.586 | 5.814 | -0.395596835 | PASS |
| 3 | Pwr1.2 | 3 | 1.524 | 5.794 | 5.586 | 5.814 | -0.343997248 | PASS |
| 4 | Pwr1.2 | 4 | 2.072 | 5.796 | 5.586 | 5.814 | -0.309597523 | PASS |

U3501 (12V to 5.7V):

| Load Regulation ( Input Voltage – 12V) for U3501 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Load Current (A) | Output Current (A) | Output Voltage (V) (avg) | Specification(V) |  | Design margin (%) | Result |
|  |  |  |  |  | Min | Max |  |  |
| 1 | Pwr1.2 | 1 | 0.486 | 5.76 | 5.586 | 5.814 | -0.92879257 | PASS |
| 2 | Pwr1.2 | 2 | 0.983 | 5.758 | 5.586 | 5.814 | -0.963192294 | PASS |
| 3 | Pwr1.2 | 3 | 1.501 | 5.743 | 5.586 | 5.814 | -1.22119023 | PASS |
| 4 | Pwr1.2 | 4 | 2.043 | 5.717 | 5.586 | 5.814 | -1.668386653 | PASS |

U4000 (12V to 3.7V):

| Load Regulation ( Input Voltage – 12V) for U4000 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Load Current (A) | Output Current (A) | Output Voltage (V) (avg) | Specification(V) |  | Design margin (%) | Result |
|  |  |  |  |  | Min | Max |  |  |
| 1 | Pwr1.2 | 1 | 0.325 | 3.737 | 3.626 | 3.774 | -0.980392157 | PASS |
| 2 | Pwr1.2 | 2 | 0.646 | 3.741 | 3.626 | 3.774 | -0.874403816 | PASS |
| 3 | Pwr1.2 | 3 | 0.983 | 3.739 | 3.626 | 3.774 | -0.927397986 | PASS |
| 4 | Pwr1.2 | 4 | 1.338 | 3.741 | 3.626 | 3.774 | -0.874403816 | PASS |

U3400 (3.7V to 1.8V):

| Load Regulation ( Input Voltage – 3.7V) for U3400 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Load Current (A) | Output Current (A) | Output Voltage (V) (avg) | Specification(V) |  | Design margin (%) | Result |
|  |  |  |  |  | Min | Max |  |  |
| 1 | Pwr1.2 | 300 | 0.15 | 1.82 | 1.773 | 1.827 | -0.383141762 | PASS |
| 2 | Pwr1.2 | 400 | 0.202 | 1.816 | 1.773 | 1.827 | -0.602079912 | PASS |
| 3 | Pwr1.2 | 500 | 0.25 | 1.815 | 1.773 | 1.827 | -0.65681445 | PASS |

NOTE:  Refer to section 8.4.7 for measurement logs of switching regulators for  Load regulation.

# Line Regulation

# Test ID

Pwr1.3

# Purpose

The purpose of this test case is to check the ability of the switching regulators to maintain its specified output voltage over changes in the input line voltage.

# Test and Measurement Method

Refer to section 3.1.3 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +11.4V DC to 12.6V DC

System/Test Load: Min-Typical-Max

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0004

# Test Results

The output voltage accuracy of switching regulators are within 2% of expected voltage under various supply input and load conditions.

U3500 (12V to 5.7V):

| Line regulation without electronic load for U3500 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply | Output | Output | Specification(V) |  | Design margin (%) | Result |
|  |  | Voltage (V) | Current (A) | Voltage (V) | Min | Max |  |  |
| 1 | Pwr1.3 | 11.4 | 0.01 | 5.79 | 5.586 | 5.814 | -0.4128 | PASS |
| 2 | Pwr1.3 | 12 | 0.01 | 5.789 | 5.586 | 5.814 | -0.43 | PASS |
| 3 | Pwr1.3 | 12.6 | 0.01 | 5.793 | 5.586 | 5.814 | -0.3612 | PASS |

| Line regulation with electronic load for U3500 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply | Load | Output | Output | Specification(V) |  | Design margin (%) | Result |
|  |  | Voltage (V) | Current (A) | Current (A) | Voltage (V) | Min | Max |  |  |
| 1 | Pwr1.3 | 11.4 | 1 | 0.519 | 5.791 | 5.586 | 5.814 | -0.3956 | PASS |
| 2 | Pwr1.3 | 11.4 | 2 | 1.047 | 5.79 | 5.586 | 5.814 | -0.4128 | PASS |
| 3 | Pwr1.3 | 11.4 | 3 | 1.6 | 5.795 | 5.586 | 5.814 | -0.3268 | PASS |
| 4 | Pwr1.3 | 11.4 | 4 | 2.184 | 5.795 | 5.586 | 5.814 | -0.3268 | PASS |
| 5 | Pwr1.3 | 12 | 1 | 0.494 | 5.788 | 5.586 | 5.814 | -0.4472 | PASS |
| 6 | Pwr1.3 | 12 | 2 | 0.998 | 5.791 | 5.586 | 5.814 | -0.3956 | PASS |
| 7 | Pwr1.3 | 12 | 3 | 1.524 | 5.794 | 5.586 | 5.814 | -0.344 | PASS |
| 8 | Pwr1.3 | 12 | 4 | 2.072 | 5.796 | 5.586 | 5.814 | -0.3096 | PASS |
| 9 | Pwr1.3 | 12.6 | 1 | 0.473 | 5.791 | 5.586 | 5.814 | -0.3956 | PASS |
| 10 | Pwr1.3 | 12.6 | 2 | 0.949 | 5.786 | 5.586 | 5.814 | -0.4816 | PASS |
| 11 | Pwr1.3 | 12.6 | 3 | 1.45 | 5.798 | 5.586 | 5.814 | -0.2752 | PASS |
| 12 | Pwr1.3 | 12.6 | 4 | 1.969 | 5.792 | 5.586 | 5.814 | -0.3784 | PASS |

U3501 (12V to 5.7V):

| Line regulation without electronic load for U3501 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply | Output | Output | Specification(V) |  | Design margin (%) | Result |
|  |  | Voltage (V) | Current (A) | Voltage (V) | Min | Max |  |  |
| 1 | Pwr1.3 | 11.4 | 0.009 | 5.781 | 5.586 | 5.814 | -0.5676 | PASS |
| 2 | Pwr1.3 | 12 | 0.009 | 5.764 | 5.586 | 5.814 | -0.85999 | PASS |
| 3 | Pwr1.3 | 12.6 | 0.009 | 5.779 | 5.586 | 5.814 | -0.602 | PASS |

| Line regulation with electronic load for U3501 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply | Load | Output | Output | Specification(V) |  | Design margin (%) | Result |
|  |  | Voltage (V) | Current (A) | Current (A) | Voltage (V) | Min | Max |  |  |
| 1 | Pwr1.3 | 11.4 | 1 | 0.512 | 5.76 | 5.586 | 5.814 | -0.92879 | PASS |
| 2 | Pwr1.3 | 11.4 | 2 | 1.038 | 5.756 | 5.586 | 5.814 | -0.99759 | PASS |
| 3 | Pwr1.3 | 11.4 | 3 | 1.585 | 5.738 | 5.586 | 5.814 | -1.30719 | PASS |
| 4 | Pwr1.3 | 11.4 | 4 | 2.153 | 5.713 | 5.586 | 5.814 | -1.73719 | PASS |
| 5 | Pwr1.3 | 12 | 1 | 0.486 | 5.76 | 5.586 | 5.814 | -0.92879 | PASS |
| 6 | Pwr1.3 | 12 | 2 | 0.983 | 5.758 | 5.586 | 5.814 | -0.96319 | PASS |
| 7 | Pwr1.3 | 12 | 3 | 1.501 | 5.743 | 5.586 | 5.814 | -1.22119 | PASS |
| 8 | Pwr1.3 | 12 | 4 | 2.043 | 5.717 | 5.586 | 5.814 | -1.66839 | PASS |
| 9 | Pwr1.3 | 12.6 | 1 | 0.465 | 5.759 | 5.586 | 5.814 | -0.94599 | PASS |
| 10 | Pwr1.3 | 12.6 | 2 | 0.935 | 5.756 | 5.586 | 5.814 | -0.99759 | PASS |
| 11 | Pwr1.3 | 12.6 | 3 | 1.43 | 5.735 | 5.586 | 5.814 | -1.35879 | PASS |
| 12 | Pwr1.3 | 12.6 | 4 | 1.942 | 5.714 | 5.586 | 5.814 | -1.71999 | PASS |

U4000 (12V to 3.7V):

| Line regulation without electronic load for U4000 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply | Output | Output | Specification(V) |  | Design margin (%) | Result |
|  |  | Voltage (V) | Current (A) | Voltage (V) | Min | Max |  |  |
| 1 | Pwr1.3 | 11.4 | 0.008 | 3.734 | 3.626 | 3.774 | -1.05988 | PASS |
| 2 | Pwr1.3 | 12 | 0.008 | 3.739 | 3.626 | 3.774 | -0.9274 | PASS |
| 3 | Pwr1.3 | 12.6 | 0.008 | 3.742 | 3.626 | 3.774 | -0.84791 | PASS |

| Line regulation with electronic load for U4000 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply | Load | Output | Output | Specification(V) |  | Design margin (%) | Result |
|  |  | Voltage (V) | Current (A) | Current (A) | Voltage (V) | Min | Max |  |  |
| 1 | Pwr1.3 | 11.4 | 1 | 0.339 | 3.737 | 3.626 | 3.774 | -0.98039 | PASS |
| 2 | Pwr1.3 | 11.4 | 2 | 0.681 | 3.739 | 3.626 | 3.774 | -0.9274 | PASS |
| 3 | Pwr1.3 | 11.4 | 3 | 1.035 | 3.738 | 3.626 | 3.774 | -0.9539 | PASS |
| 4 | Pwr1.3 | 11.4 | 4 | 1.402 | 3.741 | 3.626 | 3.774 | -0.8744 | PASS |
| 5 | Pwr1.3 | 12 | 1 | 0.325 | 3.737 | 3.626 | 3.774 | -0.98039 | PASS |
| 6 | Pwr1.3 | 12 | 2 | 0.646 | 3.741 | 3.626 | 3.774 | -0.8744 | PASS |
| 7 | Pwr1.3 | 12 | 3 | 0.983 | 3.739 | 3.626 | 3.774 | -0.9274 | PASS |
| 8 | Pwr1.3 | 12 | 4 | 1.338 | 3.741 | 3.626 | 3.774 | -0.8744 | PASS |
| 9 | Pwr1.3 | 12.6 | 1 | 0.309 | 3.741 | 3.626 | 3.774 | -0.8744 | PASS |
| 10 | Pwr1.3 | 12.6 | 2 | 0.618 | 3.737 | 3.626 | 3.774 | -0.98039 | PASS |
| 11 | Pwr1.3 | 12.6 | 3 | 0.936 | 3.737 | 3.626 | 3.774 | -0.98039 | PASS |
| 12 | Pwr1.3 | 12.6 | 4 | 1.274 | 3.739 | 3.626 | 3.774 | -0.9274 | PASS |

U3400 (3.7V to 1.8V):

| Line regulation without electronic load for U3400 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply | Output | Output | Specification(V) |  | Design margin (%) | Result |
|  |  | Voltage (V) | Current (A) | Voltage (V) | Min | Max |  |  |
| 1 | Pwr1.3 | 3.6 | 0.04 | 1.82 | 1.773 | 1.827 | -0.38314 | PASS |
| 2 | Pwr1.3 | 3.7 | 0.04 | 1.82 | 1.773 | 1.827 | -0.38314 | PASS |
| 3 | Pwr1.3 | 3.8 | 0.04 | 1.82 | 1.773 | 1.827 | -0.38314 | PASS |

| Line regulation with electronic load for U3400 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply | Load | Output | Output | Specification(V) |  | Design margin (%) | Result |
|  |  | Voltage (V) | Current (A) | Current (A) | Voltage (V) | Min | Max |  |  |
| 1 | Pwr1.3 | 3.6 | 300 | 0.153 | 1.82 | 1.773 | 1.827 | -0.38314 | PASS |
| 2 | Pwr1.3 | 3.6 | 400 | 0.207 | 1.817 | 1.773 | 1.827 | -0.54735 | PASS |
| 3 | Pwr1.3 | 3.6 | 500 | 0.261 | 1.81 | 1.773 | 1.827 | -0.93049 | PASS |
| 4 | Pwr1.3 | 3.7 | 300 | 0.15 | 1.82 | 1.773 | 1.827 | -0.38314 | PASS |
| 5 | Pwr1.3 | 3.7 | 400 | 0.202 | 1.816 | 1.773 | 1.827 | -0.60208 | PASS |
| 6 | Pwr1.3 | 3.7 | 500 | 0.25 | 1.815 | 1.773 | 1.827 | -0.65681 | PASS |
| 7 | Pwr1.3 | 3.8 | 300 | 0.147 | 1.82 | 1.773 | 1.827 | -0.38314 | PASS |
| 8 | Pwr1.3 | 3.8 | 400 | 0.198 | 1.818 | 1.773 | 1.827 | -0.49261 | PASS |
| 9 | Pwr1.3 | 3.8 | 500 | 0.248 | 1.817 | 1.773 | 1.827 | -0.54735 | PASS |

NOTE:  Refer to section 8.4.7 for measurement logs of switching regulators for line regulation.

# Ripple Noise

# Test ID

Pwr1.4

# Purpose

The purpose of this test case is to check the maximum peak-to-peak ripple voltage of switching regulators output under full load condition and typical input voltage.

# Test and Measurement Method

Refer to section 3.1.4 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage -12V DC

System/Test Load: Full

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0004

# Test Results

The maximum peak-to-peak ripple voltage measured is found to be less than 10%   (as per the LT8640IUDC specification) of the output voltage.

U3500 (12V to 5.7V):

| Ripple Measurement for U3500 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply Voltage (V) | Load Current (A) | Ripple Voltage (mV) | Specification(mV) |  | Design Margin (%) | Result |
|  |  |  |  |  | Min | Max |  |  |
| 1 | Pwr1.4 | 12 | 4 | 8.8 | 0 | 10 | -12 | PASS |

U3501 (12V to 5.7V):

| Ripple Measurement for U3501 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply Voltage (V) | Load Current (A) | Ripple Voltage (mV) | Specification(mV) |  | Design Margin (%) | Result |
|  |  |  |  |  | Min | Max |  |  |
| 1 | Pwr1.4 | 12 | 4 | 2.4 | 0 | 10 | -76 | PASS |

U4000 (12V to 3.7V):

| Ripple Measurement for U4000 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply Voltage (V) | Load Current (A) | Ripple Voltage (mV) | Specification(mV) |  | Design Margin (%) | Result |
|  |  |  |  |  | Min | Max |  |  |
| 1 | Pwr1.4 | 12 | 4 | 9.6 | 0 | 10 | -4 | PASS |

U3400 (3.7V to 1.8V):

| Ripple Measurement for U3400 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | Supply Voltage (V) | Load Current (A) | Ripple Voltage (mV) | Specification(mV) |  | Design Margin (%) | Result |
|  |  |  |  |  | Min | Max |  |  |
| 1 | Pwr1.4 | 3.7 | 500 | 8.12 | 0 | 10 | -18.8 | PASS |

# Measurement logs

The detailed analysis report with waveform captured for each of the Switching regulators function test cases executed is embed in the excel document attached herewith

# Voltage Control

# Test ID

Pwr1.5

# Purpose

The purpose of this test case is to check the Voltage Output with respect to Enable on Switching Regulators.

# Test and Measurement Method

Refer to section 3.1.1 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage -12V DC

System/Test Load: Full

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0004

# Test Results

PASS - Test case is replica of Voltage accuracy,refer to section 8.1.

# Voltage accuracy

# Test ID

Pwr1.6

# Purpose

The purpose of the test case is to measure the Output voltage of all LDO’s and to ensure that these voltages are in specified limits.

# Test and Measurement Method

Refer to section 3.1.5 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Full

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

| Voltage Accuracy for LDO’s |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | LDO | Supply Voltage (V) | Output Voltage (V) | Expected Voltage (V) | Deviation (%) | Output voltage accuracy (%) | Pass/Fail |
| 1 | Pwr1.6 | U3700 (TPS7A8300) | 5.7 | 5.04 | 5 | 0.8 | 1 | PASS |
| 2 |  | U3701 (TPS7A8300) | 5.7 | 5.066 | 5 | 1.32 | 1 | FAIL |
| 3 |  | U3600 (TPS7A8300) | 5.7 | 5.077 | 5 | 1.54 | 1 | FAIL |
| 4 |  | U3601 (TPS7A8300) | 5.7 | 5.03 | 5 | 0.6 | 1 | PASS |
| 5 |  | U3900 (TPS7A8300) | 5.7 | 5.059 | 5 | 1.18 | 1 | FAIL |
| 6 |  | U3901 (TPS7A8300) | 5.7 | 5.04 | 5 | 0.8 | 1 | PASS |
| 7 |  | U3800 (TPS7A8300) | 5.7 | 5.082 | 5 | 1.64 | 1 | FAIL |
| 8 |  | U4100 (TPS7A8300) | 3.7 | 3.295 | 3.3 | 0.151515 | 1 | PASS |
| 9 |  | U3801 (TPS7A8300) | 5.7 | 5.076 | 5 | 1.52 | 1 | FAIL |
| 10 |  | U3401 (ADP1755ACPZ-R7) | 1.8 | 1.303 | 1.3 | 0.230769 | 2 | PASS |
| 11 |  | U3402 (ADP1755ACPZ-R7) | 1.8 | 1.301 | 1.3 | 0.076923 | 2 | PASS |
| 12 |  | U4001 (TPS7A8001) | 5.7 | 5.01 | 5 | 0.2 | 3 | PASS |

Resolution for failure:

Change the tolerance of feed back resistors of LDO’s to 1%.

Results table after changing feedback resistors tolerance of LDO’s to 1%

| Voltage Accuracy for LDO’s |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sl. No. | Test case No. | LDO | Supply Voltage (V) | Output Voltage (V) | Expected Voltage (V) | Deviation (%) | Output voltage accuracy (%) | Result |
| 1 | Pwr1.6 | U3700 (TPS7A8300) | 5.7 | 5.04 | 5 | 0.8 | 1 | PASS |
| 2 |  | U3701 (TPS7A8300) | 5.7 | 4.97 | 5 | 0.87 | 1 | PASS |
| 3 |  | U3600 (TPS7A8300) | 5.7 | 4.97 | 5 | 0.87 | 1 | PASS |
| 4 |  | U3601 (TPS7A8300) | 5.7 | 5.03 | 5 | 0.6 | 1 | PASS |
| 5 |  | U3900 (TPS7A8300) | 5.7 | 4.96 | 5 | 0.87 | 1 | PASS |
| 6 |  | U3901 (TPS7A8300) | 5.7 | 5.04 | 5 | 0.8 | 1 | PASS |
| 7 |  | U3800 (TPS7A8300) | 5.7 | 4.99 | 5 | 0.87 | 1 | PASS |
| 8 |  | U4100 (TPS7A8300) | 3.7 | 3.295 | 3.3 | 0.151515 | 1 | PASS |
| 9 |  | U3801 (TPS7A8300) | 5.7 | 4.96 | 5 | 0.87 | 1 | PASS |
| 10 |  | U3401 (ADP1755ACPZ-R7) | 1.8 | 1.303 | 1.3 | 0.230769 | 2 | PASS |
| 11 |  | U3402 (ADP1755ACPZ-R7) | 1.8 | 1.301 | 1.3 | 0.076923 | 2 | PASS |
| 12 |  | U4001 (TPS7A8001) | 5.7 | 5.01 | 5 | 0.2 | 3 | PASS |

# Voltage Control

# Test ID

Pwr1.7

# Purpose

The purpose of this test case is to check the Voltage Output with respect to Enable on LDO’s.

# Test and Measurement Method

Refer to section 3.1.5 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage -12V DC

System/Test Load: Full

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0004

# Test Results

PASS - Test case is replica of Voltage accuracy ,refer to section 8.6.

# Current consumption

# Test ID

Pwr1.8

# Purpose

The purpose of the test case is to measure the board current consumption through current sensing IC.

# Test and Measurement Method

Refer to section 3.1.6 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Full

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0011

# Test Results

| RF INA226 SENSORS |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FPGA |  |  |  | CH1 |  |  |  | CH2 |  |  |  |  |
| SPEC | BUS_VOLT | MARGIN | CURRENT | SPEC | VOLT | MARGIN | CURRENT | SPEC | VOLT | MARGIN | CURRENT | RESULT |
| (mA) | (mV) | (mA) | (mA) | (mA ) | (mV) | (mA) | (mA) | (mA) | (mV) | (mA) | (mA) |  |
| <100 | 12006 | 15 | 85 | <2000 | 5666 | 372 | 1628 | <1800 | 5660 | 79 | 1721 | PASS |
| <100 | 12008 | 15 | 85 | <2000 | 5667 | 373 | 1627 | <1800 | 5660 | 84 | 1716 | PASS |
| <100 | 12006 | 15 | 85 | <2000 | 5667 | 369 | 1631 | <1800 | 5662 | 92 | 1708 | PASS |
| <100 | 12007 | 15 | 85 | <2000 | 5667 | 374 | 1626 | <1800 | 5662 | 97 | 1703 | PASS |
| <100 | 12010 | 14 | 86 | <2000 | 5667 | 180 | 1820 | <1800 | 5662 | 102 | 1698 | PASS |
| <100 | 12007 | 12 | 88 | <2000 | 5668 | 382 | 1618 | <1800 | 5662 | 108 | 1692 | PASS |
| <100 | 12010 | 10 | 90 | <2000 | 5667 | 355 | 1645 | <1800 | 5663 | 110 | 1690 | PASS |
| <100 | 12003 | 13 | 87 | <2000 | 5667 | 388 | 1612 | <1800 | 5662 | 99 | 1701 | PASS |
| <100 | 12011 | 12 | 88 | <2000 | 5668 | 389 | 1611 | <1800 | 5665 | 120 | 1680 | PASS |
| <100 | 12011 | 12 | 88 | <2000 | 5667 | 389 | 1611 | <1800 | 5665 | 125 | 1675 | PASS |
| <100 | 12012 | 10 | 90 | <2000 | 5668 | 397 | 1603 | <1800 | 5665 | 124 | 1676 | PASS |
| <100 | 12015 | 12 | 88 | <2000 | 5668 | 200 | 1800 | <1800 | 5666 | 128 | 1672 | PASS |
| <100 | 12015 | 10 | 90 | <2000 | 5670 | 394 | 1606 | <1800 | 5666 | 134 | 1666 | PASS |
| <100 | 12013 | 12 | 88 | <2000 | 5670 | 395 | 1605 | <1800 | 5666 | 134 | 1666 | PASS |
| <100 | 12012 | 10 | 90 | <2000 | 5670 | 398 | 1602 | <1800 | 5666 | 140 | 1660 | PASS |
| <100 | 12013 | 10 | 90 | <2000 | 5670 | 397 | 1603 | <1800 | 5666 | 139 | 1661 | PASS |
| <100 | 12012 | 10 | 90 | <2000 | 5671 | 399 | 1601 | <1800 | 5667 | 142 | 1658 | PASS |
| <100 | 12015 | 12 | 88 | <2000 | 5670 | 404 | 1596 | <1800 | 5665 | 143 | 1657 | PASS |
| <100 | 12015 | 13 | 87 | <2000 | 5671 | 278 | 1722 | <1800 | 5666 | 148 | 1652 | PASS |

# Clock

# Clock output level

# Test ID

CLK 1.1

# Purpose

The purpose of this test case is to verify if the PLL is locked and to check the Clock output level.

# Test and Measurement Method

Refer to section 4.1.1 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Reference input-From signal generator (For system measurements reference input will be from GBC/Sync board)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0013

# Test Results

| Clock Amplitude |  |  |  |
| --- | --- | --- | --- |
| Specification | Result | Margin | Pass/Fail |
| <1.3V(p-p) | 1V(p-p) | 0.3V | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Frequency and Frequency accuracy

# Test ID

CLK 1.2 & 1.3

# Purpose

The purpose of this test case is to verify if the frequency is within acceptable accuracy.

# Test and Measurement Method

Refer to section 4.1.2 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

All measurements are done with reference from signal generator, loop filter values are updated on the board.

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0013

# Test Results

GSM system specification is <±0.05ppm (±2Hz), since the measurement is only for RF_SDR Board without Sync board, We have a tighter spec of <±0.025ppm (±1Hz),

| Frequency Accuracy |  |  |  |
| --- | --- | --- | --- |
| Specification | Output Result(Hz) | Margin | Pass/Fail |
| <±0.025ppm(±1Hz) | -0.084 | 0.916 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Phase noise

# Test ID

CLK 1.4

# Purpose

The purpose of this test case is to verify that the phase noise of the clock is within acceptable limits

# Test and Measurement Method

Refer to section 4.1.3 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

All measurements are done with reference from signal generator, loop filter values are updated on the board.

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0013

# Test Results

| Phase Noise Measurement |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Frequency Offset | Phase Noise dBc/Hz |  | margin | Pass/Fail | Phase noise measurement dBc/Hz |
|  | Specification | Result |  |  |  |
| 10Hz | <-88 | -73.08 | -14.92 | FAIL | -82.9921 |
| 100Hz | <-115 | -93.28 | -21.72 | PASS | -115.5(Marker is on spurious) |
| 1kHz | <-138 | -104.89 | -33.11 | PASS | -140.3678 |
| 10kHz | <-145 | -109.49 | -35.51 | PASS | -151.1543 |
| 100kHz | <-150 | -118.03 | -31.97 | PASS | -155.1105 |
| 1MHz | <-152 | -138.62 | -13.38 | PASS | -155.7002 |

Resolution for failure:

Measurements were taken with reference input from signal generator having accuracy of 5Hz. In actual scenario input reference is from SYNC board which is in lock with 1PPS GPS signal,with this input as per design we will meet requirements at 10Hz.

# Test and Measurement Logs

# Lock time

# Test ID

CLK 1.5

# Purpose

The purpose of the test is to verify the maximum time taken for the PLL to settle to certain frequency and accuracy.

# Test and Measurement Method

Refer to section 4.1.4 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0013

# Test Results

| Lock Time |  |  |  |
| --- | --- | --- | --- |
| Specification | Measured | Margin | Result |
| 2 ms | 900.9 us | 1099.1 us | PASS |

NOTE: specification is from GSM frequency hoping parameter

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Clock Duty Cycle

# Test ID

CLK 1.6

# Purpose

The purpose of this test case is to verify whether the duty cycle of the clock signal satisfies the minimum required specifications

# Test and Measurement Method

Refer to section 4.1.5 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Reference input-From signal generator (For system measurements reference input will be from GBC/Sync board)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0020

# Test Results

| Test | Measuring Point | Measuring Criteria | Observation | Specification |  | Design Margin (%) | Test result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |  |
| Clock Duty Cycle |  |  |  |  |  |  |  |
| SIN_CLK_BUFF_OUT3 | R22.2 | Duty Cycle (%) | 47.73 | 25 | NA | -90.92 | PASS |

NOTE: The Max value of clock duty cycle is not mentioned in the FPGA datasheet. Hence, mentioned as NA.

# Test and Measurement Logs

# Jitter

# Test ID

CLK 1.7

# Purpose

The purpose of this test case is to verify if the period jitter of the clock signal is within the expected limit.

# Test and Measurement Method

Refer to section 4.1.6 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Reference input-From signal generator (For system measurements reference input will be from GBC/Sync board)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0020

# Test Results

| Test | Measuring Point | Measuring Criteria | Observation | Specification |  | Design Margin (%) | Test result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |  |
| Clock Period Jitter |  |  |  |  |  |  |  |
| SIN_CLK_BUFF_OUT3 | R22.2 | Period Jitter (ns) | 0.075 | NA | 5 | -98.50 | PASS |

NOTE: The Min. value of clock period jitter is not mentioned in FPGA datasheet. Hence, mentioned as NA.

# Test and Measurement Logs

# Clock Duty Cycle

# Test ID

CLK 1.8

# Purpose

The purpose of this test case is to verify whether the duty cycle of the clock signal satisfies the minimum required specifications

# Test and Measurement Method

Refer to section 4.1.7 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Reference input-From signal generator (For system measurements reference input will be from GBC/Sync board)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0020

# Test Results

| Test | Measuring Point | Measuring Criteria | Observation | Specification |  | Design Margin (%) | Test result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |  |
| Clock Duty Cycle |  |  |  |  |  |  |  |
| CAT_CLKOUT_FPGA | U9.B2 | Duty Cycle (%) | 48.47 | 25 | NA | -93.88 | PASS |

NOTE: The Max value of clock duty cycle is not mentioned in the FPGA datasheet. Hence, mentioned as NA.

# Test and Measurement Logs

# Jitter

# Test ID

CLK 1.9

# Purpose

The purpose of this test case is to verify if the period jitter of the clock signal is within the expected limit.

# Test and Measurement Method

Refer to section 4.1.8 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Reference input-From signal generator (For system measurements reference input will be from GBC/Sync board)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0020

# Test Results

| Test | Measuring Point | Measuring Criteria | Observation | Specification |  | Design Margin (%) | Test result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |  |
| Clock Period Jitter |  |  |  |  |  |  |  |
| CAT_CLKOUT_FPGA | U9.B2 | Period Jitter (ns) | 0.175 | NA | 5 | -96.50 | PASS |

NOTE: The Min. value of clock period jitter is not mentioned in FPGA datasheet. Hence, mentioned as NA.

# Test and Measurement Logs

# FPGA

# Artix – 7 –Boot configuration

# Test ID

FPGA 1.1.1

# Purpose

The purpose of the test case is to verify the boot configuration of Artix – 7 FPGA.

# Test and Measurement Method

Refer to section 5.1.1 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

Artix – 7 FPGA booted up successfully and was functioning as required.

# Test and Measurement Logs

The snapshot of boot configuration of Artix – 7 FPGA is attached below.

NOTE: This test case is a functional test. Hence, no specification table.

# Artix – 7 –Power sequence

# Test ID

FPGA 1.1.2

# Purpose

The purpose of this test case is to verify the power sequence of Artix – 7 FPGA.

# Test and Measurement Method

Refer to section 5.1.2 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

|  | Artix_7_Power_On_Sequence |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | Specification (Expected sequence) | Logic analyzer bits | Measurement Points | Measured sequence | Design Margin | Pass/Fail |
| 1 | 1P8V _FX3 | D0 | C276 | 1P8V _FX3 | NA | PASS |
| 2 | 1P2V _FX3 | D6 | C272 | 1P2V _FX3 |  |  |
| 3 | VCCINT+VCCBRAM | D4 | C245 | VCCINT+VCCBRAM |  |  |
| 4 | VCCAUX18 | D1 | C256 | VCCAUX18 |  |  |
| 5 | 1.8VD_FPGA | D2 | C251 | 1.8VD_FPGA |  |  |
| 6 | 3.3VD_FPGA | D3 | C255 | 3.3VD_FPGA |  |  |

# Test and Measurement Logs

# FX3 – SPI –Electrical validation

# Test ID

FPGA 1.2.1

# Purpose

The purpose of the test case is to verify the electrical characteristics of SPI interface of FX3.

# Test and Measurement Method

Refer to section 5.1.3 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

| Test | Measuring Point | Measuring Criteria | Observation | Specification |  | Design Margin (%) |  | Test result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |  |  |
| FX3 – SPI |  |  |  |  |  |  |  |  |
| FX3_SCLK | U9.V19 | VIL (max) (V) | 0.3 | -0.3 | 0.63 | 200.00 | PASS |  |
|  |  | VIH (min) (V) | 1.5 | 1.17 | 2.1 | -28.21 | PASS |  |
|  |  | Minimum High time (ns) | 22.6 | 2.5 | NA | -804.00 | PASS |  |
|  |  | Minimum Low time (ns) | 23.1 | 2.5 | NA | -824.00 | PASS |  |
|  |  | Frequency (MHz) | 20.16 | 0 | 100 | -79.84 | PASS |  |
| FX3_MOSI | U9.R22 | VIL (max) (V) | 0 | -0.3 | 0.63 | 100.00 | PASS |  |
|  |  | VIH (min) (V) | 1.8 | 1.17 | 2.1 | -14.29 | PASS |  |
|  |  | Minimum High time (ns) | 47.8 | 2.5 | NA | -1812.00 | PASS |  |
|  |  | Minimum Low time (ns) | 97.2 | 2.5 | NA | -3788.00 | PASS |  |

NOTE: The Max. value of Minimum High time and Minimum Low time is not mentioned in FPGA datasheet. Hence, mentioned as NA.

# Test and Measurement Logs

# FX3 – SPI –Functional validation

# Test ID

FPGA 1.2.2

# Purpose

The purpose of the test case is to validate the functioning of the SPI interface of FX3.

# Test and Measurement Method

Refer to section 5.1.4 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage – 18V

System load – Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

FX3 is able to read and write registers of FPGA and the functioning of the SPI      interface is validated.

# Test and Measurement Logs

The snapshot for the functional validation of FX3 – SPI signals is attached below.

NOTE: This test case is a functional test. Hence, no specification table.

# AD9361 – SPI-Electrical validation/Signal integrity

# Test ID

FPGA 1.3.1

# Purpose

The purpose of the test case is to verify the electrical characteristics of SPI interface of AD9361 transceiver

# Test and Measurement Method

Refer to section 5.1.5 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

| Test | Measuring Point | Measuring Criteria | Observation | Specification |  | Design Margin (%) | Test result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |  |
| AD9361 – SPI (EV) |  |  |  |  |  |  |  |
| CAT_SCLK | U9.C2 | VIL (max) (V) | -0.02 | 0 | 0.36 | -105.56 | FAIL |
|  |  | VIH (min) (V) | 1.82 | 1.44 | 1.8 | 1.11 | FAIL |
|  |  | Frequency (MHz) | 1 | 0 | 50 | -98.00 | PASS |
| CAT_MOSI | U9.A1 | VIL (max) (V) | 0.02 | 0 | 0.36 | -94.44 | PASS |
|  |  | VIH (min) (V) | 1.78 | 1.44 | 1.8 | -1.11 | PASS |
| CAT_MISO | U9.B1 | VIL (max) (V) | -0.02 | -0.3 | 0.63 | 93.33 | PASS |
|  |  | VIH (min) (V) | 1.9 | 1.17 | 2.10 | -9.52 | PASS |

Resolution for failure:

The voltage levels of  CAT_SCLK clock signal exceeds beyond the specified Min. and Max. values. A series resistor has been included in the path of the signal in Rev. C to resolve this issue.

# Test and Measurement Logs

Signal Integrity

# Purpose

The purpose of the test case is to verify the signal integrity characteristics of SPI interface of AD9361 transceiver.

# Test and Measurement Method

Refer to section 5.1.5 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document.

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

| Test | Measuring Point | Measuring Criteria | Observation | Specification |  | Design Margin (%) | Test result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |  |
| CAT_SCLK | U9.C2 | Positive Overshoot (V) | 0.645 | 0 | 0.18 | 258.33 | FAIL |
|  |  | Negative Overshoot (V) | 0.528 | 0 | 0.18 | 193.33 | FAIL |
| CAT_MOSI | U9.A1 | Positive Overshoot (V) | 0.388 | 0 | 0.18 | 115.56 | FAIL |
|  |  | Negative Overshoot (V) | 0.511 | 0 | 0.18 | 183.89 | FAIL |
|  |  | Data Setup time (ns) | 10 | 2 | NA | -400.00 | PASS |
|  |  | Data Hold time (ns) | 2000 | 1 | 1000 | 100.00 | PASS |
| CAT_MISO | U9.B1 | Positive Overshoot (V) | 0.36 | 0 | 0.18 | 100 | FAIL |
|  |  | Negative Overshoot (V) | 0.26 | 0 | 0.18 | 44.44 | FAIL |
|  |  | Data Setup time (ns) | 984 | 2.44 | NA | -40227.87 | PASS |
|  |  | Data Hold time (ns) | 4 | 0.62 | 1000 | -99.60 | PASS |

NOTE: The Max. value of Data Setup time and Data Hold time is not mentioned in the FPGA datasheet. Hence, mentioned as NA.

Resolution for failure:

The positive and negative overshoot of the signals, namely, CAT_SCLK, CAT_MOSI, and CAT_MISO exceed beyond the specified Max. value of overshoot. A series resistor has been included in the path of the signal in Rev. C to resolve this issue.

# Test and Measurement Logs

# AD9361 – SPI-Functional validation

# Test ID

FPGA 1.3.2

# Purpose

The purpose of the test case is to validate the functioning of the SPI interface of AD9361 transceiver.

# Test and Measurement Method

Refer to section 5.1.6 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document.

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage – 18V

System load – Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

The functioning of the SPI interface between AD9361 transceiver and Artix – 7 FPGA has been validated.

# Test and Measurement Logs

The snapshot for the functional validation of AD9361 – SPI signals is attached below.

NOTE: This test case is a functional test. Hence, no specification table.

# FX3 – GPIF Control – Electrical validation

# Test ID

FPGA 1.4.1

# Purpose

The purpose of the test case is to verify the electrical characteristics of control signals of FX3 – GPIF.

# Test and Measurement Method

Refer to section 5.1.7 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

| Test | Measuring Point | Measuring Criteria | Observation | Specification |  | Design Margin (%) | Test result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |  |
| FX3 – GPIF (Control) |  |  |  |  |  |  |  |
| GPIF_CTL3 | U9.G15 | VIL (max) (V) | 0 | -0.3 | 0.63 | 100.00 | PASS |
|  |  | VIH (min) (V) | 1.8 | 1.17 | 2.1 | -14.29 | PASS |
|  |  | Minimum High Time (ns) | 18000 | 2.5 | 19750 | -8.86 | PASS |
|  |  | Minimum Low Time (ns) | 1750 | 2.5 | 19750 | -91.14 | PASS |
| GPIF_CTL12 | U9.G13 | VIL (max) (V) | 0 | -0.3 | 0.63 | 100.00 | PASS |
|  |  | VIH (min) (V) | 1.8 | 1.17 | 2.1 | -14.29 | PASS |
|  |  | Minimum High Time (ns) | 200 | 2.5 | 1200 | -83.33 | PASS |
|  |  | Minimum Low Time (ns) | 1000 | 2.5 | 1200 | -16.67 | PASS |

# Test and Measurement Logs

# FX3 – GPIF Control – Functional validation

# Test ID

FPGA 1.4.2

# Purpose

The purpose of the test case is to validate the functioning of the control signals of FX3-GPIF.

# Test and Measurement Method

Refer to section 5.1.8 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage – 18V

System load – Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

The functioning of the control signals from FX3 to Artix – 7 FPGA has been      validated.

# Test and Measurement Logs

The snapshot for the functional validation of FX3 – GPIF control signals is attached below.

NOTE: This test case is a functional test. Hence, no specification table.

# FX3 – GPIF Data – Electrical validation

# Test ID

FPGA 1.5.1

# Purpose

The purpose of the test case is to verify the electrical characteristics of data signals of FX3 – GPIF.

# Test and Measurement Method

Refer to section 5.1.9 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

| Test | Measuring Point | Measuring Criteria | Observation | Specification |  | Design Margin (%) | Test result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |  |
| FX3 – GPIF (Data) |  |  |  |  |  |  |  |
| GPIF_D04 | U9.K21 | VIL (max) (V) | 0 | -0.3 | 0.63 | 100.00 | PASS |
|  |  | VIH (min) (V) | 1.88 | 1.17 | 2.1 | -10.48 | PASS |
|  |  | Minimum High Time (ns) | 10 | 2.5 | NA | -300.00 | PASS |
|  |  | Minimum Low Time (ns) | 10 | 2.5 | NA | -300.00 | PASS |
| GPIF_D19 | U9.L18 | VIL (max) (V) | 0 | -0.3 | 0.63 | 100.00 | PASS |
|  |  | VIH (min) (V) | 1.8 | 1.17 | 2.1 | -14.29 | PASS |
|  |  | Minimum High Time (ns) | 210 | 2.5 | NA | -8300.00 | PASS |
|  |  | Minimum Low Time (ns) | 45 | 2.5 | NA | -1700.00 | PASS |
| GPIF_D29 | U9.J17 | VIL (max) (V) | 0 | -0.3 | 0.63 | 100.00 | PASS |
|  |  | VIH (min) (V) | 1.84 | 1.17 | 2.1 | -12.38 | PASS |
|  |  | Minimum High Time (ns) | 145 | 2.5 | NA | -5700.00 | PASS |
|  |  | Minimum Low Time (ns) | 95 | 2.5 | NA | -3700.00 | PASS |
| GPIF_D31 | U9.L15 | VIL (max) (V) | 0 | -0.3 | 0.63 | 100.00 | PASS |
|  |  | VIH (min) (V) | 1.8 | 1.17 | 2.1 | -14.29 | PASS |
|  |  | Minimum High Time (ns) | 44 | 2.5 | NA | -1660.00 | PASS |
|  |  | Minimum Low Time (ns) | 44 | 2.5 | NA | -1660.00 | PASS |

NOTE: The Max. value of Min. High time and Min. Low time is not                      mentioned in the FPGA datasheet. Hence, mentioned as NA.

# Test and Measurement Logs

# FX3 – GPIF Data – Functional validation

# Test ID

FPGA 1.5.2

# Purpose

The purpose of the test case is to validate the functioning of the data signals of FX3 – GPIF.

# Test and Measurement Method

Refer to section 5.1.10 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage – 18V

System load – Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

FX3 is able to send data to FPGA and the functioning of the data signals has been validated.

# Test and Measurement Logs

The snapshot for the functional validation of FX3 – GPIF data signals is attached below.

NOTE: This test case is a functional test. Hence, no specification table.

# AD9361 – Control- Electrical validation

# Test ID

FPGA 1.6.1

# Purpose

The purpose of the test case is to validate the electrical characteristics of the control signals of AD9361 transceiver.

# Test and Measurement Method

Refer to section 5.1.11 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document.

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage – 18V

System load – Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

| Test | Measuring Point | Measuring Criteria | Observation | Specification |  | Design Margin (%) | Test result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |  |
| AD9361 (Control) |  |  |  |  |  |  |  |
| CODEC_CTRL_OUT2 | U9.U21 | VIL (max) (V) | -0.04 | -0.3 | 0.63 | 86.67 | PASS |
|  |  | VIH (min) (V) | 1.8 | 1.17 | 2.1 | -14.29 | PASS |
| CODEC_CTRL_OUT3 | U9.P19 | VIL (max) (V) | -0.04 | -0.3 | 0.63 | 86.67 | PASS |
|  |  | VIH (min) (V) | 1.8 | 1.17 | 2.1 | -14.29 | PASS |

# Test and Measurement Logs

# AD9361 – Control- Functional validation

# Test ID

FPGA 1.6.2

# Purpose

The purpose of the test case is to validate the functioning of AD9361 control signals.

# Test and Measurement Method

Refer to section 5.1.12 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage – 18V

System load – Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

The functioning of the control signals from AD9361 Transceiver to Artix – 7 FPGA    has been validated.

# Test and Measurement Logs

The snapshot for the functional validation of AD9361 control signals is attached below.

NOTE: This test case is a functional test. Hence, no specification table.

# AD9361 – Data- Electrical validation

# Test ID

FPGA 1.7.1

# Purpose

The purpose of the test case is to verify the electrical characteristics of data signals of AD9361 transceiver.

# Test and Measurement Method

Refer to section 5.1.13 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

| Test | Measuring Point | Measuring Criteria | Observation | Specification |  | Design Margin (%) | Test result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |  |
| AD9361 (Data) |  |  |  |  |  |  |  |
| CODEC_D1 | U9.AB12 | VIL (max) (V) | 0.01 | 0 | 0.36 | -97.22 | PASS |
|  |  | VIH (min) (V) | 1.73 | 1.44 | 1.8 | -3.89 | PASS |
|  |  | Rx Data Delay (DATA_CLK to Data Outputs) (ns) | 10.4 | 0 | 1.5 | 593.33 | FAIL |
|  |  | Rx Data Delay (DATA_CLK to Rx_FRAME) (ns) | 7.6 | 0 | 1 | 660.00 | FAIL |
| CODEC_D10 | U9.W12 | VIL (max) (V) | 0 | 0 | 0.36 | -100.00 | PASS |
|  |  | VIH (min) (V) | 1.74 | 1.44 | 1.8 | -3.33 | PASS |
|  |  | Rx Data Delay (DATA_CLK to Data Outputs) (ns) | 11 | 0 | 1.5 | 633.33 | FAIL |
|  |  | Rx Data Delay (DATA_CLK to Rx_FRAME) (ns) | 6.8 | 0 | 1 | 580.00 | FAIL |
| CODEC_D18 | U9.AA15 | VIL (max) (V) | -0.01 | 0 | 0.36 | -102.78 | PASS |
|  |  | VIH (min) (V) | 1.79 | 1.44 | 1.8 | -0.56 | PASS |
|  |  | Tx Data Setup Time (ns) | 5.9 | 0 | 1.5 | 293.33 | FAIL |
|  |  | Tx Data Hold Time (ns) | 9.3 | 0 | 1 | 830.00 | FAIL |
| CODEC_D20 | U9.Y13 | VIL (max) (V) | -0.02 | 0 | 0.36 | -105.56 | PASS |
|  |  | VIH (min) (V) | 1.8 | 1.44 | 1.8 | 0.00 | PASS |
|  |  | Tx Data Setup Time (ns) | 7.8 | 0 | 1.5 | 420.00 | FAIL |
|  |  | Tx Data Hold Time (ns) | 8 | 0 | 1 | 700.00 | FAIL |

Resolution for failure:

The Rx data delay (DATA_CLK to Data Outputs), Rx Data delay (DATA_CLK to Rx Frame), Tx Data Setup time, and Tx Data Hold time exceed beyond the specified Max. value.

Series resistors are included in the path of the signal and all the data lines are length matched in Rev. C.

# Test and Measurement Logs

# AD9361 – Data- Functional validation

# Test ID

FPGA 1.7.2

# Purpose

The purpose of the test case is to validate the functioning of AD9361 data signals.

# Test and Measurement Method

Refer to section 5.1.14 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage – 18V

System load – Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

AD9361 was able to send and receive data from FPGA and the functioning of the data signals has been validated.

# Test and Measurement Logs

The snapshot for the functional validation of AD9361 data signals is attached below.

NOTE: This test case is a functional test. Hence, no specification table.

# FX3

# FX3 (CYUSB3014)-Configuration

# Test ID

FX3 1.1

# Purpose

The purpose of this test case is to verify the configuration of FX3.

# Test and Measurement Method

Refer to section 6.1.1 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

The snapshot for FX3 configuration is attached below.

# EEPROM (24LC256) – I2C –Electrical validation

# Test ID

FX3 1.2.1

# Purpose

The purpose of the test case is to verify the electrical characteristics of I2C interface of the serial EEPROM.

# Test and Measurement Method

Refer to section 6.1.2 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

| Test | Measuring Point | Measuring Criteria | Observation | Specification |  | Design Margin (%) | Test result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |  |
| EEPROM – I2C |  |  |  |  |  |  |  |
| FX3_SCL | R61.2 | VIL (max) (V) | -0.06 | -0.5 | 0.36 | 88.00 | PASS |
|  |  | VIH (min) (V) | 1.8 | 1.26 | 2.3 | -21.74 | PASS |
|  |  | Rise time (ns) | 132 | 20 | 1000 | -86.80 | PASS |
|  |  | Fall time (ns) | 32 | 6.54 | 300 | -89.33 | PASS |
|  |  | Frequency (KHz) | 371.7 | 0 | 400 | -7.08 | PASS |
| FX3_SDA | R60.2 | VIL (max) (V) | -0.06 | -0.5 | 0.36 | 88.00 | PASS |
|  |  | VIH (min) (V) | 1.8 | 1.26 | 2.3 | -21.74 | PASS |
|  |  | Rise time (ns) | 100 | 20 | 1000 | -90.00 | PASS |
|  |  | Fall time (ns) | 33 | 6.54 | 300 | -89.00 | PASS |

# Test and Measurement Logs

# EEPROM (24LC256) – I2C –Functional validation

# Test ID

FX3 1.2.2

# Purpose

The purpose of the test case is to validate the functioning of I2C interface of the serial EEPROM.

# Test and Measurement Method

Refer to section 6.1.3 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage – 18V

System load – Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0022

# Test Results

FX3 was able to read /write data from EEPROM and the functioning of the I2C interface has been validated.

# Test and Measurement Logs

The snapshot for the functional validation of EEPROM – I2C signals is attached below.

NOTE: This test case is a functional test. Hence, no specification table.

# Functional validation of Debug USB Switch – USB2.0 from FX3

# Test ID

FX3 1.3.1

# Purpose

The purpose of the test case is to validate USB 2.0 through Debug USB Switch.

# Test and Measurement Method

Refer to section 6.1.4 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

The functional validation of USB 2.0 through Debug USB Switch is verified.

# Test and Measurement Logs

The snapshot of functional validation of USB 2.0 in Debug USB Switch are attached.

NOTE: This test case is a functional test. Hence, no specification table.

# Functional validation of Debug USB Switch – USB3.0 from FX3

# Test ID

FX3 1.4.1

# Purpose

The purpose of the test case is to validate USB 3.0 through Debug USB Switch.

# Test and Measurement Method

Refer to section 6.1.5 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

The functional validation of USB 3.0 through Debug USB Switch is verified.

# Test and Measurement Logs

The snapshot of functional validation of USB 3.0 in Debug USB Switch are attached here.

NOTE: This test case is a functional test. Hence, no specification table.

# RF/Transceiver      (AD9361) – Pipe1

# Maximum Output Power from AD9361- Pipe 1

# Test ID

TRX 1.1

# Purpose

The purpose of this test case is to check maximum power that is possible from AD9361 transceiver.

# Test and Measurement Method

Refer to section 7.1.1 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1622LIFE1SDR0003

# Test Results

| Attenuation setting from UHD code for AD9361 (dB) |  | CH1 Maximum Output Power at Balun (dBm) |  |  | Min(dBm) |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Specification | GSM-900 |  |  |  | Max(dBm) | Margin (dBm) |  |
|  |  | B | M | T |  |  |  |  |
| 0 | >-5dBm | -1.7 | -1.7 | -1.8 | -1.8 | -1.7 | 3.2 | PASS |

| Attenuation setting from UHD code for AD9361 (dB) |  | CH1 Maximum Output Power at Balun (dBm) |  |  | Min(dBm) |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Specification | DCS-1800 |  |  |  | Max (dBm) | Margin in(dBm) |  |
|  |  | B | M | T |  |  |  |  |
| 0 | >-5dBm | -3.1 | -3.1 | -3.2 | -3.2 | -3.1 | 1.8 | PASS |

NOTE:  Pig tail cable losses are taken into account during setup calibration.

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Transmit Power Control from AD9361-Pipe1

# Test ID

TRX 1.2

# Purpose

The purpose of this test case is to control Transmit power from AD9361 transceiver.

# Test and Measurement Method

Refer to section 7.1.2 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1622LIFE1SDR0003

# Test Results

| ATTENUATION SETTING FROM UHD CODE FOR AD9361 | CH1 OUTPUT POWER CONTROL |  |  |  | MIN |  | MARGIN IN (dBm) | RESULT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATION | GSM-900 |  |  |  | MAX |  |  |
|  |  | B | M | T |  |  |  |  |
| Output power level in dBm with 0dB attenuation | >-5dBm | -1.7 | -1.7 | -1.8 | -1.8 | -1.7 | 3.2 | PASS |
| Output power level in dBm with 10dB attenuation | >-15dBm | -11.5 | -11 | -11.5 | -11.5 | -11 | 3.5 | PASS |
| Attenuation in dB with 10dB atten setting in AD9361 | 10dB(+/- 2dB) | 9.8 | 9.3 | 9.7 | 9.3 | 9.8 | 1.3 | PASS |
| Output power level in dBm with 20dB attenuation | >-25dBm | -21.4 | -21 | -21.3 | -21.4 | -21 | 3.6 | PASS |
| Attenuation in dB with 20dB atten setting in AD9361 | 20dB(+/- 2dB) | 19.7 | 19.3 | 19.5 | 19.3 | 19.7 | 0.8 | PASS |

| ATTENUATION SETTING FROM UHD CODE FOR AD9361 | CH1 OUTPUT POWER CONTROL |  |  |  | MIN |  | MARGIN IN (dBm) | RESULT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATION | DCS-1800 |  |  |  | MAX |  |  |
|  |  | B | M | T |  |  |  |  |
| Output power level in dBm with 0dB attenuation | >-5dBm | -3.1 | -3.1 | -3.2 | -3.2 | -3.1 | 1.8 | PASS |
| Output power level in dBm with 10dB attenuation | >-15dBm | -12.4 | -12.5 | -12.7 | -12.7 | -12.4 | 2.6 | PASS |
| Attenuation in dB with 10dB atten setting in AD9361 | 10dB(+/- 2dB) 12dB< Atten>8dB | 9.3 | 9.4 | 9.5 | 9.3 | 9.5 | 1.3 | PASS |
| Output power level in dBm with 20dB attenuation | >-25dBm | -21.9 | -22.1 | -22.2 | -22.2 | -21.9 | 2.8 | PASS |
| Attenuation in dB with 20dB atten setting in AD9361 | 20dB(+/- 2dB) 22dB< Atten>18dB | 18.8 | 19 | 19 | 18.8 | 19 | 0.8 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Modulation Accuracy –TRx – Pipe 1

# Test ID

TRX_1.3

# Purpose

The purpose of this test case is, Phase error and EVM are fundamental parameters used in GSM to characterize modulation accuracy. These measurements reveal much about a transmitter’s performance. Poor phase error or EVM indicates a problem with the I/Q baseband generator, filters, modulator or amplifier in the transmitter circuitry.

# Test and Measurement Method

Refer to section 7.1.3 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document.

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1622LIFE1SDR0003

# Test Results

| CH1_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | GSM 900 |  |  |  |  |  |  |
|  |  |  | RMS(deg) |  |  | Min (deg) | Max (deg) | Margin (deg) |  |
|  | RMS(deg) | RMS(deg) | B | M | T |  |  |  |  |
| GMSK | <3.6 | <5 | 0.39 | 0.4 | 0.44 | 0.39 | 0.44 | 3.16 | PASS |

| CH1_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | GSM 900 |  |  |  |  |  |  |
|  |  |  | PEAK(deg) |  |  | Min (deg) | Max (deg) | Margin (deg) |  |
|  | < PEAK(deg) | < PEAK(deg) | B | M | T |  |  |  |  |
| GMSK | <14.2 | <20 | 1.02 | 0.98 | 1.23 | 0.98 | 1.23 | 12.97 | PASS |

| CH1_Mean Frequency Error |  |  |  |  |  | Min (Hz) | Max (Hz) | Margin (Hz) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification(R&D) | Specification(normal) | GSM 900 |  |  |  |  |  |  |
|  | ppm / Hz | ppm / Hz | B(Hz) | M(Hz) | T(Hz) |  |  |  |  |
| GMSK | < 0.03/±27Hz | < 0.05/±45Hz | 0.12 | -0.48 | -0.45 | -0.48 | 0.12 | 26.52 | PASS |

| CH1_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | DCS 1800 |  |  |  |  |  |  |
|  |  |  | RMS(deg) |  |  | Min (deg) | Max (deg) | Margin (deg) |  |
|  | < RMS(deg) | < RMS(deg) | B | M | T |  |  |  |  |
| GMSK | <3.6 | <5 | 0.4 | 0.47 | 0.44 | 0.4 | 0.47 | 3.13 | PASS |

| CH1_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | DCS 1800 |  |  |  |  |  |  |
|  |  |  | PEAK(deg) |  |  | Min (deg) | Max (deg) | Margin (deg) |  |
|  | < PEAK(deg) | < PEAK(deg) | B | M | T |  |  |  |  |
| GMSK | <14.2 | <20 | 1.08 | 1.38 | 1.44 | 1.08 | 1.44 | 12.76 | PASS |

| CH1_Mean Frequency Error |  |  |  |  |  | Min (Hz) | Max (Hz) | Margin (Hz) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification(R&D) | Specification(normal) | DCS 1800 |  |  |  |  |  |  |
|  | ppm / Hz | ppm / Hz | B(Hz) | M(Hz) | T(Hz) |  |  |  |  |
| GMSK | < 0.03/±54Hz | < 0.05/±90Hz | -0.44 | -0.43 | -0.18 | -0.44 | -0.18 | 53.56 | PASS |

# Test and Measurement Logs

# Output RF Spectrum- i) Adjacent channel power-TRx Pipe 1

# Test ID

TRX 1.6

# Purpose

The purpose of this test case is measure adjacent channel power, the modulation process in a transmitter causes the continuous wave (CW) Carrier to spread spectrally. The “spectrum due to modulation and wideband noise” measurement is used to ensure that modulation process does not cause excessive spectral spread. If it did, other users who are operating on different frequencies would experience interference. The measurement of spectrum due to modulation and wideband noise can be thought of as an adjacent channel power (ACP).

# Test and Measurement Method

Refer to section 7.1.4 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Signal output level: 0dBm

# DUT Sample Information

RF-SDR Board Serial Number – WZ1622LIFE1SDR0003

# Test Results

| CH1_spectrum due to modulation |  |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  |  | GSM 900 Result |  |  |  |  |  |  |  |  |  |
|  |  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin in (dB) |  |
| offset frequency | < dBc | RBW KHz | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 100KHz | 0.5 | 30 | -9.15 | -9.67 | -8.61 | -9.06 | -9.14 | -9.66 | -9.67 | -8.61 | 8.11 | PASS |
| 200KHz | -30 | 30 | -38.66 | -37.65 | -38.08 | -37.04 | -38.58 | -37.59 | -38.66 | -37.04 | 7.04 | PASS |
| 250KHz | -33 | 30 | -40.28 | -41.5 | -40.68 | -42.1 | -40.72 | -41.7 | -42.1 | -40.28 | 7.28 | PASS |
| 400KHz | -60 | 30 | -63.69 | -63.18 | -62.71 | -63.11 | -62.85 | -62.31 | -63.69 | -62.31 | 2.31 | PASS |
| 600KHz to 1200KHz | -60 | 30 | -70.97 | -71.06 | -69.82 | -69.89 | -69.61 | -69.69 | -71.06 | -69.61 | 9.61 | PASS |
| 1200KHz to 1800KHz | -63 | 30 | -75.15 | -75.66 | -74.35 | -74.28 | -74.15 | -75.11 | -75.66 | -74.15 | 11.15 | PASS |
| 1800KHz to 6000KHz | -65 | 100 | -74.52 | -74.08 | -71.13 | -73.91 | -74.05 | -74.49 | -74.52 | -71.13 | 6.13 | PASS |

| CH1_spectrum due to modulation |  |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  |  | DCS 1800 Result |  |  |  |  |  |  |  |  |  |
|  |  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin in (dB) |  |
| offset frequency | < dBc | RBW KHz | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 100KHz | 0.5 | 30 | -11.61 | -4.54 | -9.12 | -9.62 | -9.13 | -9.63 | -11.61 | -4.54 | 4.04 | PASS |
| 200KHz | -30 | 30 | -35 | -36.31 | -38.49 | -37.6 | -38.57 | -37.5 | -38.57 | -35 | 5 | PASS |
| 250KHz | -33 | 30 | -38.88 | -40.6 | -40.66 | -42.34 | -39.72 | -41.54 | -42.34 | -38.88 | 5.88 | PASS |
| 400KHz | -60 | 30 | -55.98 | -56.19 | -59.67 | -59.56 | -58.75 | -59.81 | -59.81 | -55.98 | -4.02 | FAIL |
| 600KHz to 1200KHz | -60 | 30 | -60.32 | -60.78 | -64.26 | -64.6 | -64.39 | -63.6 | -64.6 | -60.32 | 4.32 | PASS |
| 1200KHz to 1800KHz | -63 | 30 | -65.73 | -66.77 | -69.97 | -69.75 | -69.8 | -70.19 | -70.19 | -65.73 | 2.73 | PASS |
| 1800KHz to 6000KHz | -65 | 100 | -67.46 | -67.61 | -71.65 | -71.49 | -72.48 | -72.23 | -72.48 | -67.46 | 2.46 | PASS |

Resolution for failure:

We are meeting spectrum due to modulation requirements with only one chain at AD9361 active. When two chains of AD9361 are active the output of one chain is not stable (not all time slots are ON only burst broadcast channel is seen).

# Test and Measurement Logs

# ii) Spectrum due to switching- TRx Pipe 1

# Test ID

TRX 1.6

# Purpose

The purpose of this test case is the GSM/EDGE transmitter’s ramp RF power rapidly. The “transmitted RF carrier power versus time” measurement is used to ensure that this process happens at the correct times and happens fast enough. However, if RF power is ramped too quickly, undesirable spectral components exist in the transmission. This measurement is used to ensure that these components are below the acceptable level.

# Test and Measurement Method

Refer to section 7.1.5 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1622LIFE1SDR0003

# Test Results

| CH1_spectrum due to switching |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  | GSM 900 Result |  |  |  |  |  |  |  |  |  |
|  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin in (dB) |  |
| offset frequency | < dBc | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 400 KHz | -57 | -57.48 | -58.23 | -57.77 | -57.99 | -57.2 | -59.51 | -59.51 | -57.2 | 0.2 | PASS |
| 600 KHz | -67 | -64.65 | -64.87 | -64.99 | -65.32 | -64.89 | -63.72 | -65.32 | -63.72 | 3.28 | PASS |
| 1200 KHz | -74 | -67.98 | -68.43 | -66.45 | -67.34 | -67.77 | -67.3 | -68.43 | -66.45 | -7.55 | FAIL |
| 1800 KHz | -74 | -73.11 | -74.64 | -70.86 | -73.6 | -74.31 | -73.71 | -74.64 | -70.86 | -3.14 | FAIL |

| CH1_spectrum due to switching |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  | DCS 1800 Result |  |  |  |  |  |  |  |  |  |
|  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin (dB) |  |
| offset frequency | < dBc | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 400 KHz | -50 | -54.16 | -55.6 | -53.11 | -52.7 | -54.78 | -54.16 | -55.6 | -52.7 | 2.7 | PASS |
| 600 KHz | -58 | -58.53 | -58.74 | -58.35 | -58.71 | -57.92 | -57.85 | -58.74 | -57.85 | -0.15 | FAIL |
| 1200 KHz | -66 | -64.02 | -63.74 | -63.7 | -62.66 | -62.9 | -62.3 | -64.02 | -62.3 | -3.7 | FAIL |
| 1800 KHz | -66 | -72.21 | -71.88 | -72.02 | -71.54 | -72.86 | -70.69 | -72.86 | -70.69 | 4.69 | PASS |

Resolution for failure:

We are meeting spectrum due to switching requirements with only one chain at AD9361 active. When two chains of AD9361 are active the output of one chain is not stable (not all time slots are ON only burst broadcast channel is seen).

# Test and Measurement Logs

# Carrier leakage - Pipe1

# Test ID

TRX 1.4

# Purpose

The purpose of this test case is to check carrier leakage that is possible from AD9361 transceiver LO.

# Test and Measurement Method

Refer to section 7.1.6 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

| Carrier Leakage at 0dB attenuation | Spec in dBc | Carrier leakage in dBc |  |  | Margin from Spec in dB | Result |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | M | T |  | PASS |
| Chain1 | -50 | -55.7 | -55.1 | -55.8 | 5.1 |  |

| Chain1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Wanted signal power in dBm |  |  | Carrier leakage including 2dB cable loss in dBm |  |  |
| Band | B | M | T | B | M | T |
| 900 | -1.7 | -1.7 | -1.8 | -57.4 | -56.8 | -57.6 |
| 1800 | -3.1 | -3.1 | -3.2 | -54.7 | -53.78 | -55.3 |

# Test and Measurement Logs

# RF/Transceiver      (AD9361) – Pipe2

# Maximum Output Power from AD9361-Pipe2

# Test ID

TRX 2.1

# Purpose

The purpose of this test case is to check maximum power that is possible from AD9361 transceiver.

# Test and Measurement Method

Refer to section 8.1.1 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1622LIFE1SDR0003

# Test Results

| Attenuation setting from UHD code for AD9361 (dB) |  | CH2 Maximum Output Power at Balun (dBm) |  |  | Min(dBm) |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Specification | GSM-900 |  |  |  | Max(dBm) | Margin in (dBm) |  |
|  |  | B | M | T |  |  |  |  |
| 0 | >-5dBm | -1.5 | -1.4 | -1.6 | -1.6 | -1.4 | 3.4 | PASS |

| Attenuation setting from UHD code for AD9361 (dB) |  | CH2 Maximum Output Power at Balun (dBm) |  |  | Min(dBm) |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Specification | DCS-1800 |  |  |  | Max(dBm) | Margin in (dBm) |  |
|  |  | B | M | T |  |  |  |  |
| 0 | >-5dBm | -2.7 | -3.1 | -3.1 | -3.1 | -2.7 | 1.9 | PASS |

NOTE:  Pig tail cable losses are taken into account during setup calibration.

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Transmit Power Control from AD9361-Pipe2

# Test ID

TRX 2.2

# Purpose

The purpose of this test case is to control Transmit power from AD9361 transceiver.

# Test and Measurement Method

Refer to section 8.1.2 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1622LIFE1SDR0003

# Test Results

| ATTENUATION SETTING FROM UHD CODE FOR AD9361 | CH2 OUTPUT POWER CONTROL |  |  |  | MIN |  | MARGIN IN (dBm) | RESULT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SPECIFICATION | GSM-900 |  |  |  | MAX |  |  |
|  |  | B | M | T |  |  |  |  |
| Output power level in dBm with 0dB attenuation | >-5dBm | -1.5 | -1.4 | -1.6 | -1.6 | -1.4 | 3.4 | PASS |
| Output power level in dBm with 10dB attenuation | >-15dBm | -11.4 | -11 | -11.5 | -11.5 | -11 | 3.5 | PASS |
| Attenuation in dB with 10dB atten setting in AD9361 | 10dB(+/- 2dB) 12dB< Atten>8dB | 9.9 | 9.6 | 9.9 | 9.6 | 9.9 | 1.4 | PASS |
| Output power level in dBm with 20dB attenuation | >-25dBm | -20.5 | -21 | -21.1 | -21.1 | -20.5 | 3.9 | PASS |
| Attenuation in dB with 20dB atten setting in AD9361 | 20dB(+/- 2dB) 12dB< Atten>8dB | 19 | 19.6 | 19.5 | 19 | 19.6 | 0.8 | PASS |

| Attenuation setting from UHD code for AD9361 | CH2 Output Power Control |  |  |  | MIN |  | MARGIN IN (dBm) | RESULT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Specification | DCS-1800 |  |  |  | MAX |  |  |
|  |  | B | M | T |  |  |  |  |
| Output power level in dBm with 0dB attenuation | >-5dBm | -2.7 | -3.1 | -3.1 | -3.1 | -2.7 | 1.9 | PASS |
| Output power level in dBm with 10dB attenuation | >-15dBm | -12.2 | -12.5 | -12.5 | -12.5 | -12.2 | 2.5 | PASS |
| Attenuation in dB with 10dB atten setting in AD9361 | 10dB(+/- 2dB) | 9.5 | 9.4 | 9.4 | 9.4 | 9.5 | 1.4 | PASS |
| Output power level in dBm with 20dB attenuation | >-25dBm | -21.7 | -21.9 | -22.1 | -22.1 | -21.7 | 2.9 | PASS |
| Attenuation in dB with 20dB atten setting in AD9361 | 20dB(+/- 2dB) | 19 | 18.8 | 19 | 18.8 | 19 | 0.8 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Modulation Accuracy –TRx – Pipe 2

# Test ID

TRX 2.3

# Purpose

The purpose of this test case is, Phase error and EVM are fundamental parameters used in GSM to characterize modulation accuracy. These measurements reveal much about a transmitter’s performance. Poor phase error or EVM indicates a problem with the I/Q baseband generator, filters, modulator or amplifier in the transmitter circuitry.

# Test and Measurement Method

Refer to section 8.1.3 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1622LIFE1SDR0003

# Test Results

| CH2_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | GSM 900 |  |  |  |  |  |  |
|  |  |  | RMS(deg) |  |  | Min (deg) | Max (deg) | Margin in (deg) |  |
|  | < RMS(deg) | < RMS(deg) | B | M | T |  |  |  |  |
| GMSK | 3.6 | 5 | 0.41 | 0.42 | 0.4 | 0.4 | 0.42 | 3.18 | PASS |

| CH2_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | GSM 900 |  |  |  |  |  |  |
|  |  |  | PEAK(deg) |  |  | Min (deg) | Max (deg) | Margin in (deg) |  |
|  | < PEAK(deg) | < PEAK(deg) | B | M | T |  |  |  |  |
| GMSK | 14.2 | 20 | 1.01 | 1.12 | 0.96 | 0.96 | 1.12 | 13.08 | PASS |

| CH2_Mean Frequency Error |  |  |  |  |  | Min (Hz) | Max (Hz) | Margin in (Hz) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification(R&D) | Specification(normal) | GSM 900 |  |  |  |  |  |  |
|  | ppm / Hz | ppm / Hz | B(Hz) | M(Hz) | T(Hz) |  |  |  |  |
| GMSK | < 0.03/±27Hz | < 0.05/±45Hz | -0.31 | -0.49 | 0.31 | -0.49 | 0.31 | 26.51 | PASS |

| CH2_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | DCS 1800 |  |  |  |  |  |  |
|  |  |  | RMS(deg) |  |  | Min (deg) | Max (deg) | Margin in (deg) |  |
|  | < RMS(deg) | < RMS(deg) | B | M | T |  |  |  |  |
| GMSK | 3.6 | 5 | 0.4 | 0.44 | 0.47 | 0.4 | 0.47 | 3.13 | PASS |

| CH2_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | DCS 1800 |  |  |  |  |  |  |
|  |  |  | PEAK(deg) |  |  | Min (deg) | Max (deg) | Margin in (deg) |  |
|  | < PEAK(deg) | < PEAK(deg) | B | M | T |  |  |  |  |
| GMSK | 14.2 | 20 | 0.97 | 1.17 | 1.24 | 0.97 | 1.24 | 12.96 | PASS |

| CH2_Mean Frequency Error |  |  |  |  |  | Min (Hz) | Max (Hz) | Margin in (Hz) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification(R&D) | Specification(normal) | DCS 1800 |  |  |  |  |  |  |
|  | ppm / Hz | ppm / Hz | B(Hz) | M(Hz) | T(Hz) |  |  |  |  |
| GMSK | < 0.03/±54Hz | < 0.05/±90Hz | 0.01 | -0.35 | -0.9 | -0.9 | 0.01 | 53.1 | PASS |

# Test and Measurement Logs

# AD9361 Local Oscillator lock detect- Pipe 1&2

# Test ID

TRX 1.5 and TRX 2.5.

# Purpose

The purpose of this test case is to verify whether AD9361 Local oscillator is locked or not.

# Test and Measurement Method

Refer to section 8.1.4 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0013

# Test Results

Pass, All lock detects from AD9361 GPIO out pins are high. (>1.8V)

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Output RF Spectrum- i) Adjacent channel power-TRx Pipe 2

# Test ID

TRX 2.6

# Purpose

The purpose of this test case is the modulation process in a transmitter causes the continuous wave (CW) Carrier to spread spectrally. The “spectrum due to modulation and wideband noise” measurement is used to ensure that modulation process does not cause excessive spectral spread. If it did, other users who are operating on different frequencies would experience interference. The measurement of spectrum due to modulation and wideband noise can be thought of as an adjacent channel power (ACP).

# Test and Measurement Method

Refer to section 8.1.5 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Signal output level: 0dBm

# DUT Sample Information

RF-SDR Board Serial Number – WZ1622LIFE1SDR0003

# Test Results

| CH2_spectrum due to modulation |  |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  |  | GSM 900 Result |  |  |  |  |  |  |  |  |  |
|  |  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin in (dB) |  |
| offset frequency | < dBc | RBW KHz | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 100KHz | 0.5 | 30 | -10.11 | -8.02 | -9.87 | -7.77 | -9.13 | -9.64 | -10.11 | -7.77 | 7.27 | PASS |
| 200KHz | -30 | 30 | -37.79 | -37.52 | -37.37 | -37.24 | -38.58 | -37.59 | -38.58 | -37.24 | 7.24 | PASS |
| 250KHz | -33 | 30 | -40.37 | -41.74 | -40.31 | -41.62 | -40.68 | -41.72 | -41.74 | -40.31 | 7.31 | PASS |
| 400KHz | -60 | 30 | -61.45 | -62.07 | -61.78 | -61.51 | -63.74 | -63.83 | -63.83 | -61.45 | 1.45 | PASS |
| 600KHz to 1200KHz | -60 | 30 | -69.26 | -68.86 | -69.07 | -68.46 | -70.02 | -69.7 | -70.02 | -68.46 | 8.46 | PASS |
| 1200KHz to 1800KHz | -63 | 30 | -73.06 | -73.15 | -72.71 | -72.72 | -73.99 | -74.2 | -74.2 | -72.71 | 9.71 | PASS |
| 1800KHz to 6000KHz | -65 | 100 | -73 | -72.7 | -72.74 | -72.67 | -73.92 | -73.9 | -73.92 | -72.67 | 7.67 | PASS |

| CH2_spectrum due to modulation |  |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  |  | DCS 1800 Result |  |  |  |  |  |  |  |  |  |
|  |  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin in (dB) |  |
| offset frequency | < dBc | RBW KHz | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 100KHz | 0.5 | 30 | -8.9 | -9.19 | -8.81 | -9.35 | -8.88 | -9.42 | -9.42 | -8.81 | 8.31 | PASS |
| 200KHz | -30 | 30 | -38.17 | -37.32 | -38.03 | -37.43 | -38.26 | -37.44 | -38.26 | -37.32 | 7.32 | PASS |
| 250KHz | -33 | 30 | -40.32 | -41.28 | -40.11 | -41.41 | -40.17 | -41.65 | -41.65 | -40.11 | 7.11 | PASS |
| 400KHz | -60 | 30 | -62.39 | -62.7 | -59.63 | -60.08 | -59.74 | -60.56 | -62.7 | -59.63 | -0.37 | FAIL |
| 600KHz to 1200KHz | -60 | 30 | -69.3 | -68.79 | -64.37 | -64.66 | -64.6 | -64.27 | -69.3 | -64.27 | 8.27 | PASS |
| 1200KHz to 1800KHz | -63 | 30 | -73.7 | -73.86 | -69.99 | -69.62 | -69.79 | -70.33 | -73.86 | -69.62 | 6.62 | PASS |
| 1800KHz to 6000KHz | -65 | 100 | -73.2 | -74 | -71.2 | -71.27 | -71.8 | -71.88 | -74 | -71.2 | 6.2 | PASS |

Resolution for failure:

We are meeting spectrum due to modulation requirements with only one chain at AD9361 active. When two chains of AD9361 are active the output of one chain is not stable (not all time slots are ON only burst broadcast channel is seen).

# Test and Measurement Logs

# ii) Spectrum due to switching- TRx Pipe 2

# Test ID

TRX 2.6

# Purpose

The purpose of this test case is the GSM/EDGE transmitter’s ramp RF power rapidly. The “transmitted RF carrier power versus time” measurement is used to ensure that this process happens at the correct times and happens fast enough. However, if RF power is ramped too quickly, undesirable spectral components exist in the transmission. This measurement is used to ensure that these components are below the acceptable level.

# Test and Measurement Method

Refer to section 8.1.6 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1622LIFE1SDR0003

# Test Results

| CH2_spectrum due to switching |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  | GSM 900 Result |  |  |  |  |  |  |  |  |  |
|  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin in (dB) |  |
| offset frequency | < dBc | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 400 KHz | -57 | -58.24 | -59.04 | -56.22 | -58.85 | -57.55 | -58.91 | -59.04 | -56.22 | -0.78 | FAIL |
| 600 KHz | -67 | -64.35 | -64.37 | -64.43 | -64.76 | -64.27 | -63.8 | -64.76 | -63.8 | -3.2 | FAIL |
| 1200 KHz | -74 | -68.15 | -67.14 | -68 | -65.92 | -68.31 | -66.22 | -68.31 | -65.92 | -8.08 | FAIL |
| 1800 KHz | -74 | -74.46 | -74.66 | -73.96 | -74.75 | -74.24 | -74.22 | -74.75 | -73.96 | -0.04 | FAIL |

| CH2_spectrum due to switching |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  | DCS 1800 Result |  |  |  |  |  |  |  |  |  |
|  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin in (dB) |  |
| offset frequency | < dBc | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 400 KHz | -50 | -58.29 | -58.52 | -53.43 | -55.64 | -54.44 | -54.8 | -58.52 | -53.43 | 3.43 | PASS |
| 600 KHz | -58 | -63.57 | -62.41 | -59.54 | -57.52 | -59.44 | -59.61 | -63.57 | -57.52 | -0.48 | FAIL |
| 1200 KHz | -66 | -67.84 | -66.74 | -63.55 | -62.83 | -63.37 | -62.63 | -67.84 | -62.63 | -3.37 | FAIL |
| 1800 KHz | -66 | -73.34 | -74.05 | -71.72 | -70.45 | -71.39 | -71.88 | -74.05 | -70.45 | 4.45 | PASS |

Resolution for failure:

We are meeting spectrum due to switching requirements with only one chain at AD9361 active. When two chains of AD9361 are active the output of one chain is not stable (not all time slots are ON only burst broadcast channel is seen).

# Test and Measurement Logs

# Carrier leakage – Pipe2

# Test ID

TRX 2.4

# Purpose

The purpose of this test case is to check carrier leakage that is possible from AD9361 transceiver LO.

# Test and Measurement Method

Refer to section 8.1.7 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

| Carrier Leakage at 0dB attenuation | Spec in dBc | Carrier leakage in dBc |  |  | Margin from Spec in dB | Result |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | M | T |  | PASS |
| Chain2 | -50 | -51.6 | -50.68 | -52.1 | 0.68 |  |

| Chain2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Wanted signal power in dBm |  |  | Carrier leakage including 2dB cable loss in dBm |  |  |
| Band | B | M | T | B | M | T |
| 900 | -1.5 | -1.4 | -1.6 | -56.15 | -56 | -55.67 |
| 1800 | -2.7 | -3.1 | -3.1 | -57.32 | -52.7 | -53.4 |

# Test and Measurement Logs

Test Result for TRX 1.7, 1.8, 2.7 and 2.8 are missing

# TX pipe – 1

# Gain-Pipe1

# Test ID

TX_P 1.1

# Purpose

The purpose of this test case is to verify and validate TX – Pipe1 gain (excluding AD9361 transceiver).

# Test and Measurement Method

Refer to section 9.1.1 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Digital Attenuator: Minimum attenuation (0dB)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1622LIFE1SDR0003

# Test Results (Rev-A)

| Band | Gain Specification (dB) | Input Signal (dBm) | Measured Output Power (dBm) |  |  | Overall Gain (dB) |  |  | Margin in (dB) | RESULT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | M | T | B | M | T |  |  |
| E-GSM-900 | 43 | -24 | 29.1 | 30.8 | 30.6 | 49.1 | 50.8 | 50.6 | 6.1 | PASS |
| GSM-850 | 43 | -24 | 30.6 | 31.2 | 30.1 | 50.6 | 51.2 | 50.1 | 7.1 | PASS |
| DCS-1800 | 43 | -16 | 26 | 27.9 | 29 | 46 | 47.9 | 49 | 3 | PASS |
| DCS-1900 | 43 | -16 | 24.5 | 25.4 | 26 | 44.5 | 45.4 | 46 | 1.5 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Attenuation and Attenuation step- TX Pipe1

# Test ID

TX_P 1.2 and TX_P 1.3

# Purpose

The purpose of this test case is to verify TX – Pipe1 digital attenuator attenuation and attenuation step (excluding Transceiver AD9361).

# Test and Measurement Method

Refer to section 9.1.2 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0008

# Test Results

| Band | I/P Power (dBm) | Attenuation (dB) | Overall power after attenuation (dBm) | Gain (dB) | Spec (dB) | Margin (dB) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) |  |
| E-GSM-900 | -22 | 0 | 29.5 | 51.5 | ≥ 43 | 8.5 | PASS |
|  |  | 0.5 | 28.9 | 50.9 | ≥ 42.5 | 8.4 |  |
|  |  | 1 | 28.4 | 50.4 | ≥ 42 | 8.4 |  |
|  |  | 2 | 27.5 | 49.5 | ≥ 41 | 8.5 |  |
|  |  | 4 | 25.5 | 47.5 | ≥ 39 | 8.5 |  |
|  |  | 8 | 21.6 | 43.6 | ≥ 35 | 8.6 |  |
|  |  | 15.5 | 14.3 | 36.3 | ≥ 27.5 | 8.8 |  |
| Band | I/P Power (dBm) | Attenuation (dB) | Overall power after attenuation (dBm) | Gain (dB) | Spec (dB) | Margin (dB) | Result |
|  |  | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) |  |
| DCS-1800 | -14 | 0 | 31.7 | 45.7 | ≥ 43 | 2.7 | PASS |
|  |  | 0.5 | 31.3 | 45.3 | ≥ 42.5 | 2.8 |  |
|  |  | 1 | 30.8 | 44.8 | ≥ 42 | 2.8 |  |
|  |  | 2 | 29.7 | 43.7 | ≥ 41 | 2.7 |  |
|  |  | 4 | 27.8 | 41.8 | ≥ 39 | 2.8 |  |
|  |  | 8 | 24 | 38 | ≥ 35 | 3 |  |
|  |  | 15.5 | 16.6 | 30.6 | ≥ 27.5 | 3.1 |  |

| Band | Attenuation Step (dB) | Gain measured (dB) | Measured Attenuation Step (dB) | Spec from datasheet (dB) | Margin (dB) | Result |
| --- | --- | --- | --- | --- | --- | --- |
|  | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) |  |
| E-GSM -900 | 0 | 51.5 | 0 | NA | NA | PASS |
|  | 0.5 | 50.9 | 0.6 | 0.35-0.65 | 0.05 | PASS |
|  | 1 | 50.4 | 1.1 | 0.85-1.15 | 0.05 | PASS |
|  | 2 | 49.5 | 2 | 1.75-2.25 | 0.25 | PASS |
|  | 4 | 47.5 | 4 | 3.75-4.25 | 0.25 | PASS |
|  | 8 | 43.6 | 7.9 | 7.5-8.5 | 0.4 | PASS |
|  | 15.5 | 36.3 | 15.2 | 15-16 | 0.2 | PASS |

| Band | Attenuation Step (dB) | Gain measured (dB) | Measured Attenuation Step (dB) | Spec from datasheet (dB) | Margin (dB) | Result |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) |  |
| DCS-1800 | 0 | 45.7 | 0 | NA | NA | PASS |
|  | 0.5 | 45.3 | 0.4 | 0.35-0.65 | 0.05 | PASS |
|  | 1 | 44.8 | 0.9 | 0.85-1.15 | 0.05 | PASS |
|  | 2 | 43.7 | 2 | 1.75-2.25 | 0.25 | PASS |
|  | 4 | 41.8 | 3.9 | 3.75-4.25 | 0.15 | PASS |
|  | 8 | 38 | 7.7 | 7.5-8.5 | 0.2 | PASS |
|  | 15.5 | 30.6 | 15.1 | 15-16 | 0.1 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Output Power- TX Pipe 1

# Test ID

TX_P 1.4

# Purpose

The purpose of this test case is to verify TX – Pipe1 output power at antenna port (excluding Transceiver AD9361).

# Test and Measurement Method

Refer to section 9.1.3 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Digital Attenuator: Minimum attenuation (0dB)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0008

# Test Results

| Band | Output power Specification (dB) | Input Signal (dBm) | Measured Output Power (dBm) |  |  | Output power Margin (dB) | RESULT |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | M | T |  |  |
| E-GSM-900 | 33 ± 2 | -24 | 31.1 | 31.8 | 31.6 | 0.1 | PASS |
| DCS-1800 | 33 ± 2 | -16 | 31.8 | 32.2 | 32.1 | 0.8 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# RF power detection – TX Pipe1

# Test ID

TX_P 1.5

# Purpose

The purpose of this test case is to verify TX – Pipe1 RF Power detection at antenna port (excluding Transceiver AD9361).

# Test and Measurement Method

Refer to section 9.1.4 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0008

# Test Results

| Power at Antenna Port (dBm) | Power at Input of Power Detector (dBm) | ADC Decimal Value | ADC Binary Value | RESULT |
| --- | --- | --- | --- | --- |
| 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) |
| 29.5 | -7.5 | 24 | 00011000 | PASS |
| 28.9 | -8.1 | 23 | 00010111 | PASS |
| 28.4 | -8.6 | 21 | 00010101 | PASS |
| 27.5 | -9.5 | 19 | 00010011 | PASS |
| 25.5 | -11.5 | 14 | 00001110 | PASS |
| 21.6 | -15.4 | 9 | 00001001 | PASS |
| 14.3 | -22.7 | 5 | 00000101 | PASS |
| Power at Antenna Port (dBm) | Power at Input of Power Detector (dBm) | ADC Decimal Value | ADC Binary Value | RESULT |
| 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) |
| 31.7 | -5.3 | 30 | 00011110 | PASS |
| 31.3 | -5.7 | 28 | 00011100 | PASS |
| 30.8 | -6.2 | 28 | 00011100 | PASS |
| 29.7 | -7.3 | 23 | 00010111 | PASS |
| 27.8 | -9.2 | 19 | 00010011 | PASS |
| 24 | -13 | 11 | 00001011 | PASS |
| 16.6 | -20.4 | 4 | 00000100 | PASS |

# TX pipe – 2

# Gain-Pipe2

# Test ID

TX_P 2.1

# Purpose

The purpose of this test case is to verify TX – Pipe2 gain (excluding Transceiver AD9361).

# Test and Measurement Method

Refer to section 10.1.1 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Digital Attenuator: Minimum attenuation (0dB)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1622LIFE1SDR0003

# Test Results (Rev-A)

| Band | Gain Specification (dB) | Input Signal (dBm) | Measured Output Power (dBm) |  |  | Overall Gain (dB) |  |  | Min Gain Margin in (dB) | RESULT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | M | T | B | M | T |  |  |
| E-GSM-900 | 43 | -24 | 30.1 | 31.8 | 31.6 | 50.1 | 51.8 | 51.6 | 7.1 | PASS |
| GSM-850 | 43 | -24 | 31.6 | 32.2 | 31.2 | 51.6 | 52.2 | 51.2 | 8.2 | PASS |
| DCS-1800 | 43 | -16 | 26.8 | 28.4 | 31.2 | 46.8 | 48.4 | 51.2 | 3.8 | PASS |
| PCS-1900 | 43 | -16 | 24.5 | 25.8 | 26.1 | 44.5 | 45.8 | 46.1 | 1.5 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Attenuation and Attenuation step- TX Pipe2

# Test ID

TX_P 2.2 and TX_P 2.3

# Purpose

The purpose of this test case is to verify TX – Pipe2 digital attenuator attenuation and attenuation step (excluding Transceiver AD9361).

# Test and Measurement Method

Refer to section 10.1.2 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0008

# Test Results

| Band | I/P Power (dBm) | Attenuation (dB) | Overall power after attenuation (dBm) | Gain(dB) | Spec (dB) | Margin(dB) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) |  |
| E-GSM-900 | -22 | 0 | 29.7 | 51.7 | ≥ 43 | 8.7 | PASS |
|  |  | 0.5 | 29.1 | 51.1 | ≥ 42.5 | 8.6 |  |
|  |  | 1 | 28.6 | 50.6 | ≥ 42 | 8.6 |  |
|  |  | 2 | 27.6 | 49.6 | ≥ 41 | 8.6 |  |
|  |  | 4 | 25.6 | 47.6 | ≥ 39 | 8.6 |  |
|  |  | 8 | 21.6 | 43.6 | ≥ 35 | 8.6 |  |
|  |  | 15.5 | 14.3 | 36.3 | ≥ 27.5 | 8.8 |  |
| Band | I/P Power (dBm) | Attenuation (dB) | Overall power after attenuation (dBm) | Gain(dB) | Spec (dB) | Margin(dB) | Result |
|  |  | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) |  |
| DCS-1800 | -14 | 0 | 32.7 | 46.7 | ≥ 43 | 3.7 | PASS |
|  |  | 0.5 | 32.3 | 46.3 | ≥ 42.5 | 3.8 |  |
|  |  | 1 | 31.9 | 45.9 | ≥ 42 | 3.9 |  |
|  |  | 2 | 30.9 | 44.9 | ≥ 41 | 3.9 |  |
|  |  | 4 | 28.9 | 42.9 | ≥ 39 | 3.9 |  |
|  |  | 8 | 25.1 | 39.1 | ≥ 35 | 4.1 |  |
|  |  | 15.5 | 17.9 | 31.6 | ≥ 27.5 | 4.4 |  |

| Band | Attenuation Step (dB) | Gain measured (dB) | Measured Attenuation Step (dB) | Spec from datasheet (dB) | Margin (dB) | Result |
| --- | --- | --- | --- | --- | --- | --- |
|  | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) |  |
| E-GSM -900 | 0 | 51.7 | 0 | NA | NA | PASS |
|  | 0.5 | 51.1 | 0.6 | 0.35-0.65 | 0.05 | PASS |
|  | 1 | 50.6 | 1.1 | 0.85-1.15 | 0.15 | PASS |
|  | 2 | 49.6 | 2.1 | 1.75-2.25 | 0.15 | PASS |
|  | 4 | 47.6 | 4.1 | 3.75-4.25 | 0.15 | PASS |
|  | 8 | 43.6 | 8.1 | 7.5-8.5 | 0.4 | PASS |
|  | 15.5 | 36.3 | 15.4 | 15-16 | 0.4 | PASS |

| Band | Attenuation Step (dB) | Gain measured (dB) | Measured Attenuation Step (dB) | Spec from datasheet (dB) | Margin (dB) | Result |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) |  |
| DCS-1800 | 0 | 46.7 | 0 | NA | NA | PASS |
|  | 0.5 | 46.3 | 0.4 | 0.35-0.65 | 0.05 | PASS |
|  | 1 | 45.9 | 0.8 | 0.85-1.15 | 0.05 | PASS |
|  | 2 | 44.9 | 1.8 | 1.75-2.25 | 0.05 | PASS |
|  | 4 | 42.9 | 3.8 | 3.75-4.25 | 0.05 | PASS |
|  | 8 | 39.1 | 7.6 | 7.5-8.5 | 0.1 | PASS |
|  | 15.5 | 31.6 | 15.1 | 15-16 | 0.1 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Output Power- TX Pipe 2

# Test ID

TX_P 2.4

# Purpose

The purpose of this test case is to verify TX – Pipe2 output power at antenna port (excluding Transceiver AD9361).

# Test and Measurement Method

Refer to section 10.1.3 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Digital Attenuator: Minimum attenuation (0dB)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0008

# Test Results

| Band | Output power Specification (dB) | Input Signal (dBm) | Measured Output Power (dBm) |  |  | Output power Margin in (dBm) | RESULT |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | M | T |  |  |
| E-GSM-900 | 33 ± 2 | -24 | 31 | 31.329 | 31.2 | 0 | PASS |
| DCS-1800 | 33 ± 2 | -16 | 31.560 | 32.690 | 32.393 | 0.5 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# RF power detection – Tx Pipe 2

# Test ID

TX_P 2.5

# Purpose

The purpose of this test case is to verify TX – Pipe 2 RF Power detection at antenna port (excluding Transceiver AD9361).

# Test and Measurement Method

Refer to section 10.1.4 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0008

# Test Results

| Power at Antenna port (dBm) | Power at Input of Power Detector (dBm) | ADC Decimal Value | ADC Binary Value | RESULT |
| --- | --- | --- | --- | --- |
| 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) |
| 29.7 | -7.3 | 24 | 00011000 | PASS |
| 29.1 | -7.9 | 23 | 00010111 | PASS |
| 28.6 | -8.4 | 22 | 00010110 | PASS |
| 27.6 | -9.4 | 19 | 00010011 | PASS |
| 25.6 | -11.4 | 15 | 00001111 | PASS |
| 21.6 | -15.4 | 9 | 00001001 | PASS |
| 14.3 | -22.7 | 2 | 00000010 | PASS |
| Power at Antenna port (dBm) | Power at Input of Power Detector (dBm) | ADC Decimal Value | ADC Binary Value | RESULT |
| 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) |
| 32.7 | -4.3 | 33 | 00100001 | PASS |
| 32.3 | -4.7 | 33 | 00100001 | PASS |
| 31.9 | -5.1 | 30 | 00011110 | PASS |
| 30.9 | -6.1 | 27 | 00011011 | PASS |
| 28.9 | -8.1 | 22 | 00010110 | PASS |
| 25.1 | -11.9 | 14 | 00001110 | PASS |
| 17.9 | -19.1 | 5 | 00000101 | PASS |

# RX pipe – 1

# Noise Figure and Gain – Rx Pipe-1

# Test ID

RX_P 1.1 and RX_P 1.2

# Purpose

The purpose of this test case is to verify Rx Pipe -1 Noise Figure and Gain for all four bands at antenna port (excluding Transceiver AD9361).

# Test and Measurement Method

Refer to section 11.1.1 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Digital Attenuator: Minimum attenuation (0dB)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0020

# Test Results for Noise Figure

| Band | Noise Figure specification (dB) | I/P Signal from Noise Source ENR in dB | Measured Noise Figure (dB) |  |  | Margin(dB) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | M | T |  |  |
| E-GSM-900 | < 7 | 15.20 | 6.9942 | 4.6031 | 9.7379 | 2.7379 | FAIL |
| GSM-850 | < 7 | 15.20 | 5.6602 | 6.8264 | 8.6216 | 1.6216 | FAIL |
| DCS-1800 | < 7 | 15.20 | 5.0506 | 4.8100 | 7.0427 | 1.0427 | FAIL |
| PCS-1900 | < 7 | 15.20 | 6.4666 | 5.074 | 8.0144 | 1.0144 | FAIL |

# Test Results for Gain

| Band | Gain specification (dB) | I/P Signal from Noise Source ENR in dB | Measured Gain (dB) |  |  | Margin in (dB) | RESULT |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | M | T |  |  |
| E-GSM-900 | ≥ 5 | 15.20 | 8.924 | 10.736 | 7.130 | 2.130 | PASS |
| GSM-850 | ≥ 5 | 15.20 | 11.117 | 10.621 | 7.787 | 2.787 | PASS |
| DCS-1800 | ≥ 5 | 15.20 | 6.542 | 6.124 | 3.995 | -1.005 | FAIL |
| PCS-1900 | ≥ 5 | 15.20 | 5.563 | 4.770 | 1.582 | -3.418 | FAIL |

Resolution for failure:

We have removed switches and changed low noise amplifier part which is having high gain in REV_C design,through which we can  improve noise figure and gain at band edges for 1800 and 1900 bands.

# Test and Measurement Logs

# Attenuation and Attenuation step- Rx Pipe1

# Test ID

RX_P 1.3 and RX_P 1.4

# Purpose

The purpose of this test case is to verify RX – Pipe1 digital attenuator attenuation and attenuation step (excluding Transceiver AD9361).

# Test and Measurement Method

Refer to section 11.1.2 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0008

# Test Results

| Band | Input Power (dBm) | Attenuation (dB) | Overall Power after Attenuation (dBm) | Gain(dB) | Spec (dB) | Margin(dB) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 902(MHz) | 902(MHz) | 902(MHz) | 902(MHz) | 902(MHz) |  |
| E-GSM-900 | -30 | 0 | -21.2 | 8.8 | ≥ 5 | 3.8 | PASS |
|  |  | 0.5 | -21.7 | 8.3 | ≥ 4.5 | 3.8 | PASS |
|  |  | 1 | -22.2 | 7.8 | ≥ 4 | 3.8 | PASS |
|  |  | 2 | -23.2 | 6.8 | ≥ 3 | 3.8 | PASS |
|  |  | 4 | -25.2 | 4.8 | ≥1 | 3.8 | PASS |
|  |  | 8 | -29.2 | 0.8 | ≥ -3 | 3.8 | PASS |
|  |  | 15.5 | -36.6 | -6.6 | ≥ -10.5 | 3.9 | PASS |

| Band | Input Power (dBm) | Attenuation (dB) | Overall Power after Attenuation (dBm) | Gain(dB) | Spec (dB) | Margin(dB) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1747(MHz) | 1747(MHz) | 1747(MHz) | 1747(MHz) | 1747(MHz) |  |
| DCS-1800 | -30 | 0 | -24.4 | 5.6 | ≥ 5 | 1.6 | PASS |
|  |  | 0.5 | -25 | 5 | ≥ 4.5 | 0.5 | PASS |
|  |  | 1 | -25.5 | 4.5 | ≥ 4 | 0.5 | PASS |
|  |  | 2 | -26.6 | 3.4 | ≥ 3 | 0.4 | PASS |
|  |  | 4 | -28.6 | 1.4 | ≥1 | 0.4 | PASS |
|  |  | 8 | -32.8 | -2.8 | ≥ -3 | 0.2 | PASS |
|  |  | 15.5 | -40.4 | -10.4 | ≥ -10.5 | 0.1 | PASS |

| Band | Attenuation Step (dB) | Gain measured (dB) | Measured Attenuation Step (dB) | Spec from datasheet (dB) | Margin (dB) | Result |
| --- | --- | --- | --- | --- | --- | --- |
|  | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) |  |
| E-GSM -900 | 0 | 8.8 | 0 | NA | NA | PASS |
|  | 0.5 | 8.3 | 0.5 | 0.35-0.65 | 0.15 | PASS |
|  | 1 | 7.8 | 1 | 0.85-1.15 | 0.15 | PASS |
|  | 2 | 6.8 | 2 | 1.75-2.25 | 0.25 | PASS |
|  | 4 | 4.8 | 4 | 3.75-4.25 | 0.25 | PASS |
|  | 8 | 0.8 | 8 | 7.5-8.5 | 0.5 | PASS |
|  | 15.5 | -6.6 | 15.4 | 15-16 | 0.4 | PASS |

| Band | Attenuation Step (dB) | Gain measured (dB) | Measured Attenuation Step (dB) | Spec from datasheet (dB) | Margin (dB) | Result |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) |  |
| DCS-1800 | 0 | 5.6 | 0 | NA | NA | PASS |
|  | 0.5 | 5 | 0.6 | 0.35-0.65 | 0.05 | PASS |
|  | 1 | 4.5 | 1.1 | 0.85-1.15 | 0.05 | PASS |
|  | 2 | 3.4 | 2.2 | 1.75-2.25 | 0.05 | PASS |
|  | 4 | 1.4 | 4.2 | 3.75-4.25 | 0.05 | PASS |
|  | 8 | -2.8 | 8.4 | 7.5-8.5 | 0.1 | PASS |
|  | 15.5 | -10.4 | 16 | 15-16 | 0 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# RX pipe -2

# Noise Figure and Gain – Rx Pipe-2

# Test ID

RX_P 2.1 and RX_P 2.2

# Purpose

The purpose of this test case is to verify Rx Pipe -2 Noise Figure and Gain for all four bands at antenna port (excluding Transceiver AD9361).

# Test and Measurement Method

Refer to section 12.1.1 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Digital Attenuator: Minimum attenuation (0dB)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0020

# Test Results for Noise Figure

| Band | Noise Figure specification (dB) | I/P Signal from Noise Source ENR in dB | Measured Noise Figure (dB) |  |  |  | RESULT |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | M | T | Margin(dB) |  |
| E-GSM-900 | < 7 | 15.20 | 6.2058 | 4.4882 | 8.6773 | 1.6773 | FAIL |
| GSM-850 | < 7 | 15.20 | 6.1122 | 10.3793 | 8.1498 | 1.1498 | FAIL |
| DCS-1800 | < 7 | 15.20 | 4.7813 | 4.6170 | 11.4487 | 4.4487 | FAIL |
| PCS-1900 | < 7 | 15.20 | 5.7172 | 5.1511 | 7.8780 | 0.878 | FAIL |

# Test Results for Gain

| Band | Gain specification (dB) | I/P Signal from Noise Source ENR in dB | Measured Gain (dB) |  |  | Margin in (dB) | RESULT |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | M | T |  |  |
| E-GSM-900 | ≥ 5 | 15.20 | 9.784 | 11.113 | 7.367 | 2.367 | PASS |
| GSM-850 | ≥ 5 | 15.20 | 11.044 | 11.338 | 8.588 | 3.588 | PASS |
| DCS-1800 | ≥ 5 | 15.20 | 6.981 | 6.187 | -2.27 | -7.27 | FAIL |
| PCS-1900 | ≥ 5 | 15.20 | 6.086 | 4.946 | 1.695 | -3.305 | FAIL |

Resolution for failure:

We have removed switches and changed low noise amplifier part which is having high gain in REV_C design,through which we can  improve noise figure and gain at band edges for 1800 and 1900 bands.

# Test and Measurement Logs

# Attenuation and Attenuation step- Rx Pipe2

# Test ID

RX_P 2.3 and RX_P 2.4

# Purpose

The purpose of this test case is to verify RX – Pipe2 digital attenuator attenuation and attenuation step (excluding Transceiver AD9361).

# Test and Measurement Method

Refer to section 12.1.2 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0008

# Test Results

| Band | Input Power (dBm) | Attenuation (dB) | Overall Power after Attenuation (dBm) | Gain(dB) | Spec (dB) | Margin(dB) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 902(MHz) | 902(MHz) | 902(MHz) | 902(MHz) | 902(MHz) |  |
| E-GSM-900 | -30 | 0 | -21.2 | 8.8 | ≥ 5 | 3.8 | PASS |
|  |  | 0.5 | -21.7 | 8.3 | ≥ 4.5 | 3.8 | PASS |
|  |  | 1 | -22.2 | 7.8 | ≥ 4 | 3.8 | PASS |
|  |  | 2 | -23.1 | 6.9 | ≥ 3 | 3.9 | PASS |
|  |  | 4 | -25.2 | 4.8 | ≥1 | 3.8 | PASS |
|  |  | 8 | -29.1 | 0.9 | ≥ -3 | 3.9 | PASS |
|  |  | 15.5 | -36.5 | -6.5 | ≥ -10.5 | 4 | PASS |

| Band | Input Power (dBm) | Attenuation (dB) | Overall Power after Attenuation (dBm) | Gain(dB) | Spec (dB) | Margin(dB) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1747(MHz) | 1747(MHz) | 1747(MHz) | 1747(MHz) | 1747(MHz) |  |
| DCS-1800 | -30 | 0 | -24 | 6 | ≥ 5 | 1.6 | PASS |
|  |  | 0.5 | -24.7 | 5.3 | ≥ 4.5 | 0.8 | PASS |
|  |  | 1 | -25.3 | 4.7 | ≥ 4 | 0.7 | PASS |
|  |  | 2 | -26.4 | 3.6 | ≥ 3 | 0.6 | PASS |
|  |  | 4 | -28.5 | 1.5 | ≥1 | 0.5 | PASS |
|  |  | 8 | -32.6 | -2.6 | ≥ -3 | 0.4 | PASS |
|  |  | 15.5 | -40.3 | -10.3 | ≥ -10.5 | 0.2 | PASS |

| Band | Attenuation Step (dB) | Gain measured (dB) | Measured Attenuation Step (dB) | Spec from datasheet (dB) | Margin (dB) | Result |
| --- | --- | --- | --- | --- | --- | --- |
|  | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) |  |
| E-GSM -900 | 0 | 8.8 | 0 | NA | NA | PASS |
|  | 0.5 | 8.3 | 0.5 | 0.35-0.65 | 0.15 | PASS |
|  | 1 | 7.8 | 1 | 0.85-1.15 | 0.15 | PASS |
|  | 2 | 6.9 | 1.9 | 1.75-2.25 | 0.15 | PASS |
|  | 4 | 4.8 | 4 | 3.75-4.25 | 0.25 | PASS |
|  | 8 | 0.9 | 7.9 | 7.5-8.5 | 0.4 | PASS |
|  | 15.5 | -6.5 | 15.3 | 15-16 | 0.3 | PASS |

| Band | Attenuation Step (dB) | Gain measured (dB) | Measured Attenuation Step (dB) | Spec from datasheet (dB) | Margin (dB) | Result |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) |  |
| DCS-1800 | 0 | 6 | -0.4 | NA | NA | PASS |
|  | 0.5 | 5.3 | 0.3 | 0.35-0.65 | 0.05 | PASS |
|  | 1 | 4.7 | 0.9 | 0.85-1.15 | 0.05 | PASS |
|  | 2 | 3.6 | 2 | 1.75-2.25 | 0.25 | PASS |
|  | 4 | 1.5 | 4.1 | 3.75-4.25 | 0.15 | PASS |
|  | 8 | -2.6 | 8.2 | 7.5-8.5 | 0.3 | PASS |
|  | 15.5 | -10.3 | 15.9 | 15-16 | 0.1 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Transmitter _Chain 1

# i) Output Power- TX Chain 1

# Test ID

TX_C 1.2

# Purpose

The purpose of this test case is to verify TX – Chain 1 output power at antenna port.

# Test and Measurement Method

Refer to section 13.1.1 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Digital Attenuator: Minimum attenuation (0dB)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

| Band | Output power Specification (dB) | Input Signal (dBm) | Measured Output Power (dBm) |  |  | Output power Margin in (dBm) | RESULT |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | M | T |  |  |
| E-GSM-900 | 33 ± 2 | -22 | 32.1 | 32.6 | 32.4 | 1.1 | PASS |
| DCS-1800 | 33 ± 2 | -14 | 31.5 | 32 | 31.6 | 0.5 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Power Vs Time – TX Chain 1

# Test ID

TX_C 1.2

# Purpose

The purpose of this test case is to verify TX – Chain 1 Power Vs Time at antenna port.

# Test and Measurement Method

Refer to section 13.1.2 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Digital Attenuator: Minimum attenuation (0dB)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

PASS

# Test and Measurement Logs

# Static power control – TX chain1

# Test ID

TX_C 1.3

# Purpose

The purpose of this test case is to verify static power control for chain1.

# Test and Measurement Method

Refer to section 13.1.3 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0008

# Test Results

| Band | Input Power (dBm) | Attenuation (dB) | Overall Power after Attenuation (dBm) | Gain(dB) | Spec (≥ dB) | Margin(dB) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 902(MHz) | 902(MHz) | 902(MHz) | 902(MHz) | 902(MHz) |  |
| E-GSM-900 | -22 | 0 | 29.5 | 51.5 | 43 | 8.5 | PASS |
|  |  | 0.5 | 28.9 | 50.9 | 42.5 | 8.4 | PASS |
|  |  | 1 | 28.4 | 50.4 | 42 | 8.4 | PASS |
|  |  | 2 | 27.5 | 49.5 | 41 | 8.5 | PASS |
|  |  | 4 | 25.5 | 47.5 | 39 | 8.5 | PASS |
|  |  | 8 | 21.6 | 43.6 | 35 | 8.6 | PASS |
|  |  | 15.5 | 14.3 | 36.3 | 27.5 | 8.8 | PASS |

| Band | Input Power (dBm) | Attenuation (dB) | Overall Power after Attenuation (dBm) | Gain(dB) | Spec (≥ dB) | Margin(dB) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1747(MHz) | 1747(MHz) | 1747(MHz) | 1747(MHz) | 1747(MHz) |  |
| DCS-1800 | -14 | 0 | 31.7 | 45.7 | 43 | 2.7 | PASS |
|  |  | 0.5 | 31.3 | 45.3 | 42.5 | 2.8 | PASS |
|  |  | 1 | 30.8 | 44.8 | 42 | 2.8 | PASS |
|  |  | 2 | 29.7 | 43.7 | 41 | 2.7 | PASS |
|  |  | 4 | 27.8 | 41.8 | 39 | 2.8 | PASS |
|  |  | 8 | 24 | 38 | 35 | 3 | PASS |
|  |  | 15.5 | 16.6 | 30.6 | 27.5 | 3.1 | PASS |

| Band | Attenuation Step (dB) | Gain measured (dB) | Measured Attenuation Step (dB) | Spec from datasheet (dB) | Margin (dB) | Result |
| --- | --- | --- | --- | --- | --- | --- |
|  | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) |  |
| E-GSM -900 | 0 | 51.5 | 0 | NA | NA | PASS |
|  | 0.5 | 50.9 | 0.6 | 0.35-0.65 | 0.05 | PASS |
|  | 1 | 50.4 | 1.1 | 0.85-1.15 | 0.05 | PASS |
|  | 2 | 49.5 | 2 | 1.75-2.25 | 0.25 | PASS |
|  | 4 | 47.5 | 4 | 3.75-4.25 | 0.25 | PASS |
|  | 8 | 43.6 | 7.9 | 7.5-8.5 | 0.4 | PASS |
|  | 15.5 | 36.3 | 15.2 | 15-16 | 0.2 | PASS |

| Band | Attenuation Step (dB) | Gain measured (dB) | Measured Attenuation Step (dB) | Spec from datasheet (dB) | Margin (dB) | Result |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) |  |
| DCS-1800 | 0 | 45.7 | 0 | NA | NA | PASS |
|  | 0.5 | 45.3 | 0.4 | 0.35-0.65 | 0.05 | PASS |
|  | 1 | 44.8 | 0.9 | 0.85-1.15 | 0.05 | PASS |
|  | 2 | 43.7 | 2 | 1.75-2.25 | 0.25 | PASS |
|  | 4 | 41.8 | 3.9 | 3.75-4.25 | 0.15 | PASS |
|  | 8 | 38 | 7.7 | 7.5-8.5 | 0.2 | PASS |
|  | 15.5 | 30.6 | 15.1 | 15-16 | 0.1 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Modulation Accuracy for TX – Chain 1

# Test ID

TX_C_1.4

# Purpose

The purpose of this test case is, Phase error and EVM are fundamental parameters used in GSM to characterize modulation accuracy. These measurements reveal much about a transmitter’s performance. Poor phase error or EVM indicates a problem with the I/Q baseband generator, filters, modulator or amplifier in the transmitter circuitry.

# Test and Measurement Method

Refer to section 13.1.4 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document.

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Digital Attenuator: Minimum attenuation (0dB)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

| CH1_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | GSM 900 |  |  |  |  |  |  |
|  |  |  | RMS(deg) |  |  | Min (deg) | Max (deg) | Margin (deg) |  |
|  | RMS(deg) | RMS(deg) | B | M | T |  |  |  |  |
| GMSK | <3.6 | <5 | 0.64 | 0.64 | 0.64 | 0.64 | 0.64 | 2.96 | PASS |

| CH1_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | GSM 900 |  |  |  |  |  |  |
|  |  |  | PEAK(deg) |  |  | Min (deg) | Max (deg) | Margin in (deg) |  |
|  | < PEAK(deg) | < PEAK(deg) | B | M | T |  |  |  |  |
| GMSK | <14.2 | <20 | 1.55 | 1.72 | 1.71 | 1.55 | 1.72 | 12.48 | PASS |

| CH1_Mean Frequency Error |  |  |  |  |  | Min (Hz) | Max (Hz) | Margin in (Hz) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification(R&D) | Specification(normal) | GSM 900 |  |  |  |  |  |  |
|  | ppm / Hz | ppm / Hz | B(Hz) | M(Hz) | T(Hz) |  |  |  |  |
| GMSK | < 0.03/±27Hz | < 0.05/±45Hz | -1.8 | -2.1 | -5.85 | -5.85 | -1.8 | 21.15 | PASS |

| CH1_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | DCS 1800 |  |  |  |  |  |  |
|  |  |  | RMS(deg) |  |  | Min (deg) | Max (deg) | Margin in (deg) |  |
|  | < RMS(deg) | < RMS(deg) | B | M | T |  |  |  |  |
| GMSK | <3.6 | <5 | 0.53 | 0.56 | 0.62 | 0.53 | 0.62 | 2.98 | PASS |

| CH1_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | DCS 1800 |  |  |  |  |  |  |
|  |  |  | PEAK(deg) |  |  | Min (deg) | Max (deg) | Margin in (deg) |  |
|  | < PEAK(deg) | < PEAK(deg) | B | M | T |  |  |  |  |
| GMSK | <14.2 | <20 | 1.33 | 1.84 | 1.68 | 1.33 | 1.84 | 12.36 | PASS |

| CH1_Mean Frequency Error |  |  |  |  |  | Min (Hz) | Max (Hz) | Margin (Hz) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification(R&D) | Specification(normal) | DCS 1800 |  |  |  |  |  |  |
|  | ppm / Hz | ppm / Hz | B(Hz) | M(Hz) | T(Hz) |  |  |  |  |
| GMSK | < 0.03/±54Hz | < 0.05/±90Hz | -1.22 | 0.38 | -1.95 | -1.95 | 0.38 | 52.05 | PASS |

# Test and Measurement Logs

# Output RF Spectrum- i) Adjacent channel power-TX Chain 1

# Test ID

TX_C 1.5

# Purpose

The purpose of this test case is measure adjacent channel power, the modulation process in a transmitter causes the continuous wave (CW) Carrier to spread spectrally. The “spectrum due to modulation and wideband noise” measurement is used to ensure that modulation process does not cause excessive spectral spread. If it did, other users who are operating on different frequencies would experience interference. The measurement of spectrum due to modulation and wideband noise can be thought of as an adjacent channel power (ACP).

# Test and Measurement Method

Refer to section 13.1.5 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

| CH1_spectrum due to modulation |  |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  |  | GSM 900 Result |  |  |  |  |  |  |  |  |  |
|  |  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin(dB) |  |
| offset frequency | < dBc | RBW KHz | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 100KHz | 0.5 | 30 | -8.94 | -9.25 | -10.39 | -6.23 | -8.89 | -8.7 | -10.39 | -6.23 | 6.73 | PASS |
| 200KHz | -30 | 30 | -38.26 | -37.31 | -36.55 | -36.35 | -37.8 | -37.11 | -38.26 | -36.35 | 6.35 | PASS |
| 250KHz | -33 | 30 | -40.37 | -41.43 | -39.51 | -40.27 | -40.42 | -41.38 | -41.43 | -39.51 | 6.51 | PASS |
| 400KHz | -60 | 30 | -57.79 | -57.48 | -54.77 | -54.88 | -57.55 | -58.41 | -58.41 | -54.77 | -5.23 | FAIL |
| 600KHz to 1200KHz | -60 | 30 | -61.96 | -62.15 | -59.46 | -59.14 | -62.2 | -62.21 | -62.21 | -59.14 | -0.86 | FAIL |
| 1200KHz to 1800KHz | -63 | 30 | -72.28 | -72.24 | -69.32 | -69.39 | -72.02 | -71.79 | -72.28 | -69.32 | 6.32 | PASS |
| 1800KHz to 6000KHz | -65 | 100 | -71.97 | -72.21 | -68.79 | -69.37 | -71.43 | -71.44 | -72.21 | -68.79 | 3.79 | PASS |

| CH1_spectrum due to modulation |  |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  |  | DCS 1800 Result |  |  |  |  |  |  |  |  |  |
|  |  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin(dB) |  |
| offset frequency | < dBc | RBW KHz | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 100KHz | 0.5 | 30 | -9.01 | -9.25 | -8.75 | -9.34 | -8.78 | -9.23 | -9.34 | -8.75 | 9.25 | PASS |
| 200KHz | -30 | 30 | -38.32 | -37.57 | -38.31 | -37.13 | -37.85 | -37.06 | -38.32 | -37.13 | 7.13 | PASS |
| 250KHz | -33 | 30 | -40.36 | -41.42 | -40.2 | -41.7 | -40.79 | -42.09 | -42.09 | -40.2 | 7.2 | PASS |
| 400KHz | -60 | 30 | -57.82 | -57.48 | -57.47 | -58.28 | -57.88 | -56.97 | -58.28 | -57.47 | -2.53 | FAIL |
| 600KHz to 1200KHz | -60 | 30 | -63 | -63.09 | -62.61 | -63.44 | -62 | -62.58 | -63.44 | -62 | 2 | PASS |
| 1200KHz to 1800KHz | -63 | 30 | -71.69 | -71.21 | -69.17 | -68.03 | -72.33 | -70.31 | -72.33 | -68.03 | 5.03 | PASS |
| 1800KHz to 6000KHz | -65 | 100 | -72.58 | -71.82 | -71.28 | -71.76 | -71.41 | -71.73 | -72.58 | -71.28 | 6.28 | PASS |

Resolution for failure:

We have seen improvement in spectrum due to modulation by changing charge pump current value in AD9361 transceiver.

# Test and Measurement Logs

# ii) Spectrum due to switching- TX Chain 1

# Test ID

TX_C 1.5

# Purpose

The purpose of this test case is the GSM/EDGE transmitter’s ramp RF power rapidly. The “transmitted RF carrier power versus time” measurement is used to ensure that this process happens at the correct times and happens fast enough. However, if RF power is ramped too quickly, undesirable spectral components exist in the transmission. This measurement is used to ensure that these components are below the acceptable level.

# Test and Measurement Method

Refer to section 13.1.6 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

| CH1_spectrum due to switching |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  | GSM 900 Result |  |  |  |  |  |  |  |  |  |
|  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin(dB) |  |
| offset frequency | < dBc | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 400 KHz | -57 | -53.19 | -53.79 | -53.63 | -52.89 | -54.56 | -53.65 | -54.56 | -52.89 | -4.11 | FAIL |
| 600 KHz | -67 | -55.75 | -58.02 | -57.94 | -56.34 | -57.11 | -57.66 | -58.02 | -55.75 | -11.25 | FAIL |
| 1200 KHz | -74 | -66.55 | -66.29 | -67.42 | -67.43 | -66.14 | -66.93 | -67.43 | -66.14 | -7.86 | FAIL |
| 1800 KHz | -74 | -72.32 | -72.52 | -73.24 | -73.17 | -73.44 | -72.68 | -73.44 | -72.32 | -1.68 | FAIL |

| CH1_spectrum due to switching |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  | DCS 1800 Result |  |  |  |  |  |  |  |  |  |
|  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin (dB) |  |
| offset frequency | < dBc | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 400 KHz | -50 | -54.88 | -54.33 | -54.56 | -53.23 | -53.09 | -52.56 | -54.88 | -53.09 | 3.09 | PASS |
| 600 KHz | -58 | -58.79 | -58.53 | -58.51 | -54.96 | -58.36 | -56.38 | -58.79 | -54.96 | -3.04 | FAIL |
| 1200 KHz | -66 | -67.14 | -65.35 | -66.51 | -64.9 | -65.24 | -66.35 | -67.14 | -64.9 | -1.1 | FAIL |
| 1800 KHz | -66 | -74.49 | -71.99 | -73.47 | -72.76 | -72.5 | -73.01 | -74.49 | -71.99 | 5.99 | PASS |

Resolution for failure:

Need software support to change raise time/fall time of each time slot.

# Test and Measurement Logs

# Spurious Emissions – TX chain1

# Test ID

TX_C 1.6

# Purpose

The purpose of this test case is to ensure GSM transmitters do not put energy into the wrong parts of the spectrum, as this would cause interference to other users of the spectrum.

# Test and Measurement Method

Refer to section 13.1.7 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

| CH1_900MHZ |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Start Frequency (MHz) | Start Frequency (MHz) | Spec (dBm) | RBW (KHz) | VBW (KHz) | Amplitude(dBm) |  |  | Max (dBm) | Margin (dB) | Result |
|  |  |  |  |  | B | M | T |  |  |  |
| 0.1 | 50 | -36 | 10 | 30 | no spur | no spur | no spur | 0 | 36 | PASS |
| 50 | 880 | -36 | 3000 | 9000 | -65.14 | -64.25 | -64.21 | -64.21 | 28.21 | PASS |
| 880 | 915 | -98 | 100 | 100 | -80.89 | -80.18 | -79.93 | -79.93 | -18.07 | FAIL |
| 915 | 920 | -36 | 100 | 300 | -81.44 | -80.2 | -82.25 | -80.2 | 44.2 | PASS |
| 920 | 923 | -36 | 30 | 90 | -74.5 | -77.5 | -83.02 | -74.5 | 38.5 | PASS |
| 925 | 960 |  |  |  |  |  |  |  |  |  |
| 962 | 965 | -36 | 30 | 90 | -77.86 | -71.84 | -67.02 | -67.02 | 31.02 | PASS |
| 965 | 970 | -36 | 100 | 300 | -76.35 | -70.3 | -67.58 | -67.58 | 31.58 | PASS |
| 970 | 980 | -36 | 300 | 900 | -73.31 | -72.32 | -66.38 | -66.38 | 30.38 | PASS |
| 989 | 990 | -36 | 1000 | 3000 | -71.22 | -68.24 | -69.14 | -68.24 | 32.24 | PASS |
| 990 | 1000 | -36 | 3000 | 9000 | -65.22 | -64.11 | -64.17 | -64.11 | 28.11 | PASS |
| 1000 | 12750 | -30 | 3000 | 9000 | -45.99 | -45 | -53 | -45 | 15 | PASS |

| CH1_1800MHZ |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Start Frequency (MHz) | Start Frequency (MHz) | Spec (dBm) | RBW (KHz) | VBW (KHz) | Amplitude(dBm) |  |  | Max (dBm) | Margin (dB) | Result |
|  |  |  |  |  | B | M | T |  |  |  |
| 0.1 | 50 | -36 | 10 | 30 | no spur | no spur | no spur | 0 | 36 | PASS |
| 50 | 1000 | -36 | 3000 | 9000 | -66.3 | -66.4 | -65.99 | -65.99 | 29.99 | PASS |
| 1000 | 1710 | -30 | 3000 | 9000 | -65 | -64.8 | -63 | -63 | 33 | PASS |
| 1710 | 1785 | -98 | 100 | 100 | -80.14 | -79.95 | -80.2 | -79.95 | -18.05 | FAIL |
| 1785 |  | -30 | 300 | 900 | -75.9 | -76.25 | -76 | -75.9 | 45.9 | PASS |
| 1795 |  | -30 | 100 | 300 | -74 | -79 | -78 | -74 | 44 | PASS |
| 1800 | 1803 | -30 | 30 | 90 | -69 | -80 | -82 | -69 | 39 | PASS |
| 1805 | 1880 |  |  |  |  |  |  |  |  |  |
| 1882 | 1885 | -30 | 30 | 90 | -82 | -73.42 | -63.51 | -63.51 | 33.51 | PASS |
| 1887 | 1890 | -30 | 100 | 300 | -76.8 | -77.3 | -65.2 | -65.2 | 35.2 | PASS |
| 1890 | 1900 | -30 | 300 | 900 | -74 | -73.63 | -60 | -60 | 30 | PASS |
| 1900 | 1910 | -30 | 1000 | 3000 | -68 | -68.28 | -62 | -62 | 32 | PASS |
| 1910 | 12750 | -30 | 3000 | 9000 | -56.76 | -65 | -61 | -56.76 | 26.76 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

Resolution for failure:

We have added band pass filter in REV_C design on TX side for better rejections in self RX band.

# Transmitter _Chain 2

# i) Output Power- TX Chain 2

# Test ID

TX_C 2.2

# Purpose

The purpose of this test case is to verify TX – Chain 2 output power at antenna port.

# Test and Measurement Method

Refer to section 14.1.1 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Digital Attenuator: Minimum attenuation (0dB)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

| Band | Output power Specification (dB) | Input Signal (dBm) | Measured Output Power (dBm) |  |  | Output power Margin (dBm) | RESULT |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | M | T |  |  |
| E-GSM-900 | 33 ± 2 | -22 | 32.7 | 33.5 | 33.4 | 1.7 | PASS |
| DCS-1800 | 33 ± 2 | -14 | 31.5 | 32.6 | 32.3 | 0.5 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Power Vs Time – TX Chain 2

# Test ID

TX_C 2.2

# Purpose

The purpose of this test case is to verify TX – Chain 2 Power Vs Time at antenna port.

# Test and Measurement Method

Refer to section 14.1.2 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Digital Attenuator: Minimum attenuation (0dB)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

PASS

# Test and Measurement Logs

# Static power control – TX chain2

# Test ID

TX_C 2.3

# Purpose

The purpose of this test case is to verify static power control for chain2.

# Test and Measurement Method

Refer to section 14.1.3 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0008

# Test Results

| Band | Input Power (dBm) | Attenuation (dB) | Overall Power after Attenuation (dBm) | Gain(dB) | Spec (≥ dB) | Margin(dB) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 902(MHz) | 902(MHz) | 902(MHz) | 902(MHz) | 902(MHz) |  |
| E-GSM-900 | -22 | 0 | 29.7 | 51.7 | 43 | 8.7 | PASS |
|  |  | 0.5 | 29.1 | 51.1 | 42.5 | 8.6 | PASS |
|  |  | 1 | 28.6 | 50.6 | 42 | 8.6 | PASS |
|  |  | 2 | 27.6 | 49.6 | 41 | 8.6 | PASS |
|  |  | 4 | 25.6 | 47.6 | 39 | 8.6 | PASS |
|  |  | 8 | 21.6 | 43.6 | 35 | 8.6 | PASS |
|  |  | 15.5 | 14.3 | 36.3 | 27.5 | 8.8 | PASS |

| Band | Input Power (dBm) | Attenuation (dB) | Overall Power after Attenuation (dBm) | Gain(dB) | Spec (≥ dB) | Margin(dB) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1747(MHz) | 1747(MHz) | 1747(MHz) | 1747(MHz) | 1747(MHz) |  |
| DCS-1800 | -14 | 0 | 32.7 | 46.7 | 43 | 3.7 | PASS |
|  |  | 0.5 | 32.3 | 46.3 | 42.5 | 3.8 | PASS |
|  |  | 1 | 31.9 | 45.9 | 42 | 3.9 | PASS |
|  |  | 2 | 30.9 | 44.9 | 41 | 3.9 | PASS |
|  |  | 4 | 28.9 | 42.9 | 39 | 3.9 | PASS |
|  |  | 8 | 25.1 | 39.1 | 35 | 4.1 | PASS |
|  |  | 15.5 | 17.7 | 31.7 | 27.5 | 4.2 | PASS |

| Band | Attenuation Step (dB) | Gain measured (dB) | Measured Attenuation Step (dB) | Spec from datasheet (dB) | Margin (dB) | Result |
| --- | --- | --- | --- | --- | --- | --- |
|  | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) | 945.2(MHz) |  |
| E-GSM -900 | 0 | 51.7 | 0 | NA | NA | PASS |
|  | 0.5 | 51.1 | 0.6 | 0.35-0.65 | 0.05 | PASS |
|  | 1 | 50.6 | 1.1 | 0.85-1.15 | 0.05 | PASS |
|  | 2 | 49.6 | 2.1 | 1.75-2.25 | 0.05 | PASS |
|  | 4 | 47.6 | 4.1 | 3.75-4.25 | 0.15 | PASS |
|  | 8 | 43.6 | 8.1 | 7.5-8.5 | 0.4 | PASS |
|  | 15.5 | 36.3 | 15.4 | 15-16 | 0.4 | PASS |

| Band | Attenuation Step (dB) | Gain measured (dB) | Measured Attenuation Step (dB) | Spec from datasheet (dB) | Margin (dB) | Result |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) | 1842.4(MHz) |  |
| DCS-1800 | 0 | 46.7 | 0 | NA | NA | PASS |
|  | 0.5 | 46.3 | 0.4 | 0.35-0.65 | 0.05 | PASS |
|  | 1 | 45.9 | 0.8 | 0.85-1.15 | 0.05 | PASS |
|  | 2 | 44.9 | 1.8 | 1.75-2.25 | 0.05 | PASS |
|  | 4 | 42.9 | 3.8 | 3.75-4.25 | 0.05 | PASS |
|  | 8 | 39.1 | 7.6 | 7.5-8.5 | 0.4 | PASS |
|  | 15.5 | 31.7 | 15 | 15-16 | 0 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

# Modulation Accuracy for TX – Chain 2

# Test ID

TX_C_2.4

# Purpose

The purpose of this test case is, Phase error and EVM are fundamental parameters used in GSM to characterize modulation accuracy. These measurements reveal much about a transmitter’s performance. Poor phase error or EVM indicates a problem with the I/Q baseband generator, filters, modulator or amplifier in the transmitter circuitry.

# Test and Measurement Method

Refer to section 14.1.4 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

Digital Attenuator: Minimum attenuation (0dB)

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

| CH2_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | GSM 900 |  |  |  |  |  |  |
|  |  |  | RMS(deg) |  |  | Min (deg) | Max (deg) | Margin (deg) |  |
|  | < RMS(deg) | < RMS(deg) | B | M | T |  |  |  |  |
| GMSK | 3.6 | 5 | 0.64 | 0.64 | 0.65 | 0.64 | 0.65 | 2.95 | PASS |

| CH2_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | GSM 900 |  |  |  |  |  |  |
|  |  |  | PEAK(deg) |  |  | Min (deg) | Max (deg) | Margin (deg) |  |
|  | < PEAK(deg) | < PEAK(deg) | B | M | T |  |  |  |  |
| GMSK | 14.2 | 20 | 1.67 | 1.51 | 1.67 | 1.51 | 1.67 | 12.53 | PASS |

| CH2_Mean Frequency Error |  |  |  |  |  | Min (Hz) | Max (Hz) | Margin (Hz) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification(R&D) | Specification(normal) | GSM 900 |  |  |  |  |  |  |
|  | ppm / Hz | ppm / Hz | B(Hz) | M(Hz) | T(Hz) |  |  |  |  |
| GMSK | < 0.03/±27Hz | < 0.05/±45Hz | 2.66 | -0.74 | -0.41 | -0.74 | 2.66 | 24.34 | PASS |

| CH2_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | DCS 1800 |  |  |  |  |  |  |
|  |  |  | RMS(deg) |  |  | Min (deg) | Max (deg) | Margin (deg) |  |
|  | < RMS(deg) | < RMS(deg) | B | M | T |  |  |  |  |
| GMSK | 3.6 | 5 | 0.51 | 0.54 | 0.54 | 0.51 | 0.54 | 3.06 | PASS |

| CH2_Phase Error |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification (R&D) | Specification (Normal) | DCS 1800 |  |  |  |  |  |  |
|  |  |  | PEAK(deg) |  |  | Min (deg) | Max (deg) | Margin (deg) |  |
|  | < PEAK(deg) | < PEAK(deg) | B | M | T |  |  |  |  |
| GMSK | 14.2 | 20 | 1.32 | 1.46 | 1.65 | 1.32 | 1.65 | 12.55 | PASS |

| CH2_Mean Frequency Error |  |  |  |  |  | Min (Hz) | Max (Hz) | Margin (Hz) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | Specification(R&D) | Specification(normal) | DCS 1800 |  |  |  |  |  |  |
|  | ppm / Hz | ppm / Hz | B(Hz) | M(Hz) | T(Hz) |  |  |  |  |
| GMSK | < 0.03/±54Hz | < 0.05/±90Hz | -0.12 | -0.92 | 0.22 | -0.92 | 0.22 | 53.08 | PASS |

# Test and Measurement Logs

# Output RF Spectrum- i) Adjacent channel power-Tx Chain 2

# Test ID

TX_C 2.5

# Purpose

The purpose of this test case is the modulation process in a transmitter causes the continuous wave (CW) Carrier to spread spectrally. The “spectrum due to modulation and wideband noise” measurement is used to ensure that modulation process does not cause excessive spectral spread. If it did, other users who are operating on different frequencies would experience interference. The measurement of spectrum due to modulation and wideband noise can be thought of as an adjacent channel power (ACP).

# Test and Measurement Method

Refer to section 14.1.5 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

| CH2_spectrum due to modulation |  |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  |  | GSM 900 Result |  |  |  |  |  |  |  |  |  |
|  |  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin(dB) |  |
| offset frequency | < dBc | RBW KHz | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 100KHz | 0.5 | 30 | -9.88 | -7.5 | -9.1 | -9.45 | -9.1 | -9.33 | -9.88 | -7.5 | 8 | PASS |
| 200KHz | -30 | 30 | -37.48 | -36.83 | -38.47 | -37.43 | -38.32 | -37.34 | -38.47 | -36.83 | 6.83 | PASS |
| 250KHz | -33 | 30 | -39.95 | -40.77 | -40.38 | -41.53 | -40.64 | -41.33 | -41.53 | -39.95 | 6.95 | PASS |
| 400KHz | -60 | 30 | -56.79 | -56.16 | -57.79 | -58.33 | -57.11 | -57.03 | -58.33 | -56.16 | -3.84 | FAIL |
| 600KHz to 1200KHz | -60 | 30 | -59.84 | -60.85 | -61.86 | -63.03 | -61.64 | -62.42 | -63.03 | -59.84 | -0.16 | FAIL |
| 1200KHz to 1800KHz | -63 | 30 | -70.83 | -70.51 | -72.94 | -73.19 | -72.1 | -71.64 | -73.19 | -70.51 | 7.51 | PASS |
| 1800KHz to 6000KHz | -65 | 100 | -70.38 | -70.34 | -72.25 | -72.17 | -71.11 | -71.35 | -72.25 | -70.34 | 5.34 | PASS |

| CH2_spectrum due to modulation |  |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  |  | DCS 1800 Result |  |  |  |  |  |  |  |  |  |
|  |  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin(dB) |  |
| offset frequency | < dBc | RBW KHz | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 100KHz | 0.5 | 30 | -8.84 | -9.07 | -9.94 | -7.9 | -9.19 | -9.53 | -9.94 | -7.9 | 8.4 | PASS |
| 200KHz | -30 | 30 | -38.18 | -37.07 | -37.61 | -37.23 | -38.36 | -36.86 | -38.36 | -37.07 | 7.07 | PASS |
| 250KHz | -33 | 30 | -40.53 | -41.69 | -39.93 | -41.28 | -40.22 | -41.62 | -41.69 | -39.93 | 6.93 | PASS |
| 400KHz | -60 | 30 | -58.2 | -57.68 | -57.84 | -57.53 | -58.15 | -57.86 | -58.2 | -57.53 | -2.47 | FAIL |
| 600KHz to 1200KHz | -60 | 30 | -63.22 | -63.21 | -62.06 | -62.14 | -63.02 | -63.62 | -63.62 | -62.06 | 2.06 | PASS |
| 1200KHz to 1800KHz | -63 | 30 | -73 | -72.15 | -71.81 | -71.12 | -72.7 | -72.44 | -73 | -71.12 | 8.12 | PASS |
| 1800KHz to 6000KHz | -65 | 100 | -72.53 | -71.36 | -70.96 | -70.79 | -71.59 | -71.97 | -72.53 | -70.79 | 5.79 | PASS |

Resolution for failure:

We have seen improvement in spectrum due to modulation by changing charge pump current value in AD9361 transceiver.

# Test and Measurement Logs

# ii) Spectrum due to switching- TX Chain 2

# Test ID

TX_C 2.5

# Purpose

The purpose of this test case is the GSM/EDGE transmitter’s ramp RF power rapidly. The “transmitted RF carrier power versus time” measurement is used to ensure that this process happens at the correct times and happens fast enough. However, if RF power is ramped too quickly, undesirable spectral components exist in the transmission. This measurement is used to ensure that these components are below the acceptable level.

# Test and Measurement Method

Refer to section 14.1.6 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

| CH2_spectrum due to switching |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  | GSM 900 Result |  |  |  |  |  |  |  |  |  |
|  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin (dB) |  |
| offset frequency | < dBc | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 400 KHz | -57 | -53.56 | -52.57 | -52.52 | -53.17 | -53.37 | -51.34 | -53.56 | -52.52 | -4.48 | FAIL |
| 600 KHz | -67 | -56.65 | -58.21 | -56.82 | -56.69 | -57.26 | -56.43 | -58.21 | -56.65 | -10.35 | FAIL |
| 1200 KHz | -74 | -66.25 | -67.5 | -68.89 | -68.5 | -67.17 | -66.97 | -68.89 | -66.25 | -7.75 | FAIL |
| 1800 KHz | -74 | -73.34 | -73.77 | -72.28 | -72.1 | -72.81 | -72.37 | -73.77 | -72.1 | -1.9 | FAIL |

| CH2_spectrum due to switching |  |  |  |  |  |  |  |  |  |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification |  | DCS 1800 Result |  |  |  |  |  |  |  |  |  |
|  |  | B(dBc) |  | M(dBc) |  | T(dBc) |  | Min (dBc) | Max (dBc) | Margin (dB) |  |
| offset frequency | < dBc | Lower | Upper | Lower | Upper | Lower | Upper |  |  |  |  |
| 400 KHz | -50 | -55.79 | -54.2 | -55.35 | -55.44 | -53.76 | -54.47 | -55.79 | -53.76 | 3.76 | PASS |
| 600 KHz | -58 | -58.92 | -57.96 | -58.23 | -59.22 | -59.4 | -58.06 | -59.4 | -57.96 | -0.04 | FAIL |
| 1200 KHz | -66 | -69.7 | -68.22 | -68.84 | -68.98 | -67.95 | -67.36 | -69.7 | -67.95 | 1.95 | PASS |
| 1800 KHz | -66 | -73.79 | -72.92 | -73.6 | -74.11 | -72.52 | -73.49 | -74.11 | -72.52 | 6.52 | PASS |

Resolution for failure:

Need software support to change raise time/fall time of each time slot.

# Test and Measurement Logs

# Spurious Emissions – TX chain2

# Test ID

TX_C 2.6

# Purpose

The purpose of this test case is to ensure GSM transmitters do not put energy into the wrong parts of the spectrum, as this would cause interference to other users of the spectrum.

# Test and Measurement Method

Refer to section 14.1.7 of Open Cellular – Connect1 Radio Frequency module with Software Defined Radio Test Specification document

# Test Condition

Ambient Temperature – 25˚C

Operating Voltage - +12V DC

System/Test Load: Typical

# DUT Sample Information

RF-SDR Board Serial Number – WZ1630LIFE2SDR0010

# Test Results

| CH2_900MHZ |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Start Frequency (MHz) | Start Frequency (MHz) | Spec (dBm) | RBW (KHz) | VBW (KHz) | Amplitude(dBm) |  |  | Max (dBm) | Margin (dB) | Result |
|  |  |  |  |  | B | M | T |  |  |  |
| 0.1 | 50 | -36 | 10 | 30 | no spur | no spur | no spur | 0 | 36 | PASS |
| 50 | 880 | -36 | 3000 | 9000 | -66 | -64.5 | -65 | -64.5 | 28.5 | PASS |
| 880 | 915 | -98 | 100 | 100 | -80.98 | -80.4 | -79.12 | -79.12 | -18.88 | FAIL |
| 915 | 920 | -36 | 100 | 300 | -78.6 | -79.8 | -80.28 | -78.6 | 42.6 | PASS |
| 920 | 923 | -36 | 30 | 90 | -73.4 | -78.6 | no spur | -73.4 | 37.4 | PASS |
| 925 | 960 |  |  |  |  |  |  |  |  |  |
| 962 | 965 | -36 | 30 | 90 | -80.46 | -70 | -66.25 | -66.25 | 30.25 | PASS |
| 965 | 970 | -36 | 100 | 300 | -74.41 | -67.27 | -66.6 | -66.6 | 30.6 | PASS |
| 970 | 980 | -36 | 300 | 900 | -75.7 | -72.6 | -67.63 | -67.63 | 31.63 | PASS |
| 989 | 990 | -36 | 1000 | 3000 | -71.3 | -68.99 | -69.25 | -68.99 | 32.99 | PASS |
| 990 | 1000 | -36 | 3000 | 9000 | -64.85 | -64.5 | -63.22 | -63.22 | 27.22 | PASS |
| 1000 | 12750 | -30 | 3000 | 9000 | -47.13 | -45.02 | -53.2 | -45.02 | 15.02 | PASS |

| CH2_1800MHZ |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Start Frequency (MHz) | Start Frequency (MHz) | Spec (dBm) | RBW (KHz) | VBW (KHz) | Amplitude(dBm) |  |  | Max (dBm) | Margin (dB) | Result |
|  |  |  |  |  | B | M | T |  |  |  |
| 0.1 | 50 | -36 | 10 | 30 | no spur | no spur | no spur | 0 | 36 | PASS |
| 50 | 1000 | -36 | 3000 | 9000 | -66.6 | -65 | -65.14 | -65 | 29 | PASS |
| 1000 | 1710 | -30 | 3000 | 9000 | -65.2 | -63 | -63.44 | -63 | 33 | PASS |
| 1710 | 1785 | -98 | 100 | 100 | -79.93 | -79.5 | -78.87 | -78.87 | -19.13 | FAIL |
| 1785 |  | -30 | 300 | 900 | -74.4 | -74.33 | -73.9 | -73.9 | 43.9 | PASS |
| 1795 |  | -30 | 100 | 300 | -73 | -78 | -77 | -73 | 43 | PASS |
| 1800 | 1803 | -30 | 30 | 90 | -69.35 | -77.95 | -83 | -69.35 | 39.35 | PASS |
| 1805 | 1880 |  |  |  |  |  |  |  |  |  |
| 1882 | 1885 | -30 | 30 | 90 | -82 | -73 | -64 | -64 | 34 | PASS |
| 1887 | 1890 | -30 | 100 | 300 | -76.8 | -76 | -62.22 | -62.22 | 32.22 | PASS |
| 1890 | 1900 | -30 | 300 | 900 | -73 | -70 | -56 | -56 | 26 | PASS |
| 1900 | 1910 | -30 | 1000 | 3000 | -69.47 | -67 | -60 | -60 | 30 | PASS |
| 1910 | 12750 | -30 | 3000 | 9000 | -57.45 | -60 | -57 | -57 | 27 | PASS |

NOTE:  Screen shots were not captured, if required we make sure that we are capturing screen shots for REV_C boards.

Resolution for failure:

We have added band pass filter in REV_C design on TX side for better rejections in self RX band.

# Revision History

| SL.no | Date | Version | Author | Comments |
| --- | --- | --- | --- | --- |
| 1 | February 9th, 2017 | 1.0 | OpenCellular Team | First Release |